WASTE MANAGEMENT

Waste management is the collection, transport, processing, recycling or disposal of waste materials. The term usually relates to materials produced by human activity, and is generally undertaken to reduce their effect on health, the environment or aesthetics. Waste management is also carried out to recover resources from it. Waste management can involve solid, liquid, gaseous or radioactive substances, with different methods and fields of expertise for each.

Waste management practices differ for developed and developing nations, for urban and rural areas, and for residential and industrial, producers. Management for non-hazardous residential and institutional waste in metropolitan areas is usually the responsibility of local government authorities, while management for non-hazardous commercial and industrial waste is usually the responsibility of the generator.

Solid Waste Disposal, disposal of normally solid or semisolid materials, resulting from human and animal activities, that are useless, unwanted, or hazardous. Solid wastes typically may be classified as follows:

Garbage: decomposable wastes from food

Rubbish: non decomposable wastes, either combustible (such as paper, wood, and cloth) or noncombustible (such as metal, glass, and ceramics)

Ashes: residues of the combustion of solid fuels

Dead animals

Sewage-treatment solids: material retained on sewage-treatment screens, settled solids, and biomass sludge

Industrial wastes: such materials as chemicals, paints, and sand

- WASTE MANAGEMENT METHODS

Waste management methods vary widely between areas for many reasons, including type of waste material, nearby land uses, and the area available.

- DISPOSAL METHODS

- LANDFILL

Disposing of waste in a landfill involves burying waste to dispose it off, and this remains a common practice in most countries. Landfills were often established in abandoned or unused quarries, mining voids or borrow pits. A properly-designed and well-managed landfill can be a hygienic and relatively inexpensive method of disposing of waste materials. Older, poorly-designed or poorly-managed landfills can create a number of adverse environmental impacts. Common byproduct of landfills is gas (mostly composed of methane and carbon dioxide), which is produced as organic waste breaks down anaerobically. This gas can create odor problems, kill surface vegetation, and is a greenhouse gas.

In a modern landfill, refuse is spread in thin layers, each of which is compacted by a bulldozer before the next is spread. When about 3 m (about 10 ft) of refuse has been laid down, it is covered by a thin layer of clean earth, which also is compacted. Pollution of surface and groundwater is minimized by lining and contouring the fill, compacting and planting the cover, selecting proper soil, diverting upland drainage, and placing wastes in sites not subject to flooding or high groundwater levels. Gases are generated in landfills through anaerobic decomposition of organic solid waste. If a significant amount of methane is present, it may be explosive; proper venting eliminates this problem.

#### INCINERATION

Incineration is a disposal method that involves combustion of waste material. Incineration and other high temperature waste treatment systems are sometimes described as "thermal treatment". Incinerators convert waste materials into heat, gas, steam, and ash. Incineration is carried out both on a small scale by individuals and on a large scale by industry. It is used to dispose of solid, liquid and gaseous waste. It is recognized as a practical method of disposing of certain hazardous waste materials (such as biological medical waste). Incineration is a controversial method of waste disposal, due to issues such as emission of gaseous pollutants.

Incineration is common in countries such as Japan where land is more scarce, as these facilities generally do not require as much area as landfills. waste-to-energy (WtE) or energy-from-waste (EfW) are broad terms for facilities that burn waste in a furnace or boiler to generate heat, steam and/or electricity. Combustion in an incinerator is not always perfect and there have been concerns about micro-pollutants in gaseous emissions from incinerator stacks. On the other hand this method produces heat that can be used as energy.

- RECYCLING METHODS

The process of extracting resources or value from waste is generally referred to as recycling, meaning to recover or reuse the material. There are a number of different methods by which waste material is recycled: the raw materials may be extracted and reprocessed, or the calorific content of the waste may be converted to electricity. New methods of recycling are being developed continuously, and are described briefly below.

- PHYSICAL REPROCESSING

The popular meaning of ‘recycling’ in most developed countries refers to the widespread collection and reuse of everyday waste materials such as empty beverage containers. These are collected and sorted into common types so that the raw materials from which the items are made can be reprocessed into new products. Material for recycling may be collected separately from general waste using dedicated bins and collection vehicles, or sorted directly from mixed waste streams.

The most common consumer products recycled include aluminum beverage cans, steel food and aerosol cans, PET bottles, glass bottles and jars, paperboard cartons, newspapers, magazines, and cardboard. Other types of plastic are also recyclable, although these are not as commonly collected. These items are usually composed of a single type of material, making them relatively easy to recycle into new products. The recycling of complex products (such as computers and electronic equipment) is more difficult, due to the additional dismantling and separation required.

- BIOLOGICAL REPROCESSING

Waste materials that are organic in nature, such as plant material, food scraps, and paper products, can be recycled using biological composting and digestion processes to decompose the organic matter. The resulting organic material is then recycled as mulch or compost for agricultural or landscaping purposes. In addition, waste gas from the process (such as methane) can be captured and used for generating electricity. The intention of biological processing in waste management is to control and accelerate the natural process of decomposition of organic matter.

There are a large variety of composting and digestion methods and technologies varying in complexity from simple home compost heaps, to industrial-scale enclosed-vessel digestion of mixed domestic waste (see Mechanical biological treatment). Methods of biological decomposition are differentiated as being aerobic or anaerobic methods, though hybrids of the two methods also exist.

- COMPOSTING

Composting operations of solid wastes include preparing refuse and degrading organic matter by aerobic microorganisms. Refuse is presorted, to remove materials that might have salvage value or cannot be composted, and is ground up to improve the efficiency of the decomposition process. The refuse is placed in long piles on the ground or deposited in mechanical systems, where it is degraded biologically to a humus with a total nitrogen, phosphorus, and potassium content of 1 to 3 percent, depending on the material being composted. After about three weeks, the product is ready for curing, blending with additives, bagging, and marketing.