Experiment No.3: To determine the kinetics of fading indicators in alkaline solutions

Apparatus: spectrophotometer; cells; beaker; pipette; sucker etc.

Required Chemicals: Sodium Hydroxide (NaOH); Phenolphthalein (C20H14O4); distilled water (H2O).

Theory:

Phenolphthalein is one of the most common acid-base indicators used to determine the end point in acid-base titrations.  It is also the active ingredient in some laxatives.  If excess base is present at the end of an acid-base titration, the pink phenolphthalein color fades if the solution is allowed to stand for a while.  This slow fading is of no consequence to the titration.  However the fading of phenolphthalein in basic solution is an excellent example of second order reaction kinetics.

The structure of the colorless form of phenolphthalein (C20H14O4) is complex and not important to this experiment.  At a pH of 8 or lower, the structure of phenolphthalein will be abbreviated with the formula H2P.  In the pH range of 8-10 both acidic protons are rapidly removed from the colorless H2P by NaOH, (OH -) to give the pink P2- ion according to the equation:

(1)H2p            (2) P-2      (3)POH-3

H2P (aq) + 2 OH-(aq) → P2-(aq) + 2 H2O (l)                                     pink                            colorless

At a pH above 10 the pink color slowly fades as P2- reacts further with NaOH, (OH -), to give the colorless POH3- ion according to the equation:

P2-(aq) + OH-(aq) → POH3-(aq)                         pink            colorless

The rate law for this second order reaction can be expressed as:

# Rate = k2 [P2-]m[NaOH]n  or Rate = k2[P2-]m[NaOH]n

The purpose of this experiment is to show that both m and n are equal to unity or that the reaction is first order in phenolphthalein and first order in NaOH (OH- ion).  In this experiment the solutions will be strongly basic with only a trace of phenolphthalein. In any of the individual runs, the NaOH concentration exceeds that of phenolphthalein by a factor of 103 to 104.  Therefore, during each run the NaOH concentration remains constant and the rate law becomes:  Rate = k1[P-2]m.  In this new rate law, k1 = k2[NaOH]n, and the reaction is said to be “pseudo lst order” in phenolphthalein, the species P2-, if m = 1.  If the reaction is indeed first order in phenolphthalein (m = 1) then a plot of ln[P2-] versus time will give a straight line with a slope equal to –k1.

In this experiment the amount of P2- in the reaction solution is determined by measuring the absorbance of 550 nm light.  If Io is the intensity of light at 550 nm entering the solution and I is the intensity of light exiting the solution, then the transmittance, T, of the solution is given as I/Io.  Transmittance is also expressed as a percentage, (I/Io) x (100%).  Frequently, the absorbance, A, rather than transmittance is used for the amount of light a solution absorbs.  Absorbance is defined by the equation A = -log (T) or A = log(lo/I).  The absorbance of a solution depends on the quantity of light absorbing species in the solution and the length of the solution the light passes through.  This relationship is known as Beer's law and is expressed mathematically as

Absorbance = ε b C                                                                                                                        Where

ε =molar absorptivity (dm-3/mol. cm) or M-1cm-1

l=path length (cm)

C=concentration (mol/dm3)

The result is that [P2-] is directly proportional to the absorbance A of the solution at 550 nm.  Thus a plot of ln(A) versus time will give a straight line if the reaction is first order in P2- and its slope will be -k1.  The rate law is then:

# Rate = k2 (Absorbance)m [NaOH]n

By measuring the reaction rate at several different concentrations of NaOH, it can be shown that the reaction is first order in NaOH and the rate constant, k2 can be evaluated.

k1 = k2[NaOH]n if n = 0, then k2 will equal k1 for all four runs. If n = 1 then k2 equals k1 divided by  [NaOH] and k1/[NaOH] will be constant for all four runs. If n = 2 then k2 equals k1 divided by [NaOH]2 and k1/[NaOH]2 will be constant for all for runs. Thus the reaction order in NaOH can be determined.

Procedure:

*0.30M stock solutions of NaOH and NaCl was prepared in distilled water.

*1% alcoholic phenolphthalein solution was prepared and diluted by ethyl alcohol using (1ml+4ml) of respective solutions.

*Different molarity solutions of NaOH was prepared in NaCl solvent by using dilution formula.

*One drop of phenolphthalein solution is added to about half of cuvette of NaOH solution and the absorbance is measured at regular interval of time.

*The reading was taken for every 30s interval for twelve minute and for dilute concentrations, interval of time was 2 min.

*Graph was plotted between time and ln(A) and slope was calculated.

*Rate of reaction was calculated by using experimental data.

Observation and Calculations:

0.30M NaOH

Molarity=massmolarmass×1V(dm3)
            0.3       =x/40.1/0.25
            mass      =1.2g

0.30M NaCl

Molarity=massmolarmass×1V(dm3)

Mass   =4.38g

| S.NO | Concentration of NaOH in Molar | V1=M2V2/M1 (ml) |
| --- | --- | --- |
| 1 | 0.24 | 0.24 *10/0.3 v1=8ml |
| 2 | 0.2 | 7 |
| 3 | 0.1 | 6 |
| 4 | 0.05 | 5 |
| 5 | 0.06 | 2 |

V2=volume of NaCl used to diute all solutions 10ml

Measurement of absorbance at 550nm for all samples of NaOH.

|  | 0.300 M NaOH |  |  |  | 0.200 M NaOH |  |
| --- | --- | --- | --- | --- | --- | --- |
| Time (min) | Absorbance (A) | ln(A) |  | Time(min) | Absorbance (A) | ln(A) |
| 0.00 | 0.6 | -0.5 |  | 0.00 | 0.6 | -0.5 |
| 0.50 | 0.46 | -0.7 |  | 1.00 | 0.46 | -0.7 |
| 1.00 | 0.39 | -0.9 |  | 2.00 | 0.39 | -0.9 |
| 1.50 | 0.33 | -1.1 |  | 3.00 | 0.27 | -1.3 |
| 2.00 | 0.29 | -1.2 |  | 4.00 | 0.23 | -1.4 |
| 2.50 | 0.21 | -1.5 |  | 5.00 | 0.21 | -1.5 |
| 3.00 | 0.20 | -1.6 |  | 6.00 | 0.20 | -1.6 |

Run #3                                                                     Run #4

|  | 0.100 M NaOH |  |  |  | 0.050 M NaOH |  |
| --- | --- | --- | --- | --- | --- | --- |
| Time (min) | Absorbance (A) | ln(A) |  | Time(min) | Absorbance (A) | ln(A) |
| 0.00 | 0.6 | -0.5 |  | 0.00 | 0.6 | -0.5 |
| 2.00 | 0.47 | -0.74 |  | 2.00 | 0.46 | -0.7 |
| 4.00 | 0.37 | -1 |  | 4.00 | 0.38 | -0.96 |
| 6.00 | 0.30 | -1.2 |  | 6.00 | 0.29 | -1.2 |
| 8.00 | 0.23 | -1.5 |  | 8.00 | 0.22 | -1.5 |
| 10.00 | 0.20 | -1.6 |  | 10.00 | 0.19 | -1.6 |
| 12.00 | 0.18 | -1.7 |  | 12.00 | 0.17 | -1.7 |

Graph No 1:

ln(A) vs. Time(min) at 550nm having –slope K1 (-0.37).

Table:

To calculate the order of reaction:

|  |  |  | A (Zero order in base) | B (1st order in base) | C (2nd order in base) |
| --- | --- | --- | --- | --- | --- |
| Run No. | [NaOH] (M) | k1 | k2 = k1/[NaOH-]0 or k2 = k1 | k2 = k1/[NaOH-] | k2 = k1/[NaOH-]2 |
| 1 | 0.3 | 0.37 | 0.37 | 1.23 | 4.11 |