Crop Water Requirement

PLANT WATER DEMAND/REQUIREMENT

- The relationship between the reference grass crop and the crop actually grown is given by the crop factor, Kc.

- The crop factor, Kc, mainly depends on:

  - the type of crop
· the growth stage of the crop
· the climate

Crop ET

  - ETc = Crop evapotranspiration (mm/day)

  - ETo = Reference crop evapotranspiration (mm/day)

  - Kc = Crop coefficient

How to Calculate Daily  irrigation Requirement

(a). Daily Crop Water Requirement 
             (Row/Field Crop)

Crop Water Requirement, mm/day

ETc 		= (ETo X Kc)

ETo = Reference Evapotranspiration rate in mm/day

- Kc = Crop Factor

(b). Daily Crop Water Requirement 
             (Orchard)

Crop Water Requirement, mm/day

ETc 		= (ETo X Kc  x Cp)

ETo = Reference Evapotranspiration rate in mm/day

- Kc = Crop Factor

- Cp = Canopy factor

Factors affecting Etc

- Climatic Factors

- Soil Water Factors

- Irrigation Method

- Cultural Practices

Irrigation Requirements

IRn = ETc - (Pe + Ge + Wb) + LRmm

Where:

  - IRn = Net irrigation requirement (mm)

  - ETc = Crop evapotranspiration (mm)

  - Pe = Effective dependable rainfall (mm)

  - Ge = Groundwater contribution from water table (mm)

  - Wb = Water stored in the soil at the beginning of each period (mm)

  - LRmm = Leaching requirement (mm)

IRRIGATION SCHEDULING

- How to Apply

  - Flood, Bed & Furrow, Sprinkler, Drip, Center pivot.

- When one should apply water.

  - Rate of water use by the crop.

  - Total available moisture.

- How Much water should be applied during irrigation.

  - The soil available moisture storage capacity.

  - The amount of available water depleted from the soil profile by crop water use.

Poor Irrigation Management

- Over and Under Irrigation.

- Reduce Crop Yield.

- Degrade Crop Quality.

- Enhance the field environment for disease/Insect/weeds epidemic.

- Increase Pumping Cost.

- Leach soluble nutrients from root zone

WHY SCHEDULING

- Each soil has different moisture holding capacity.

- Plants differ in their abilities to withdraw water from soil.

- Plants differ in their water use rate.

- Water quality (salinity) may influence water availability to plants.

- Irrigation System may have inherent characteristics to restrict frequency, rate or duration of water application.

- Certain irrigation Practices may be needed for reasons other than meeting evapo-transpiration requirements such as:

      - frost control,

      - softening of soil crust,

      - softening of clods

      - incorporation of herbicides in the soil

      - Nutrient application

- External factor Such as:

      - labour scheduling,

- availability of water,

FACTORS EFFECTING IRRIGATION SCHEDULING

- Availability of water in the soil

- Water needs of crop

- How water is needed by the crop/plants

- Rainfall

- Output of irrigation system

PHYSICAL PROPERTIES OF SOILS

| Soil Texture | Field Capacity (Fc) | Permanent Wilting Point (PWP) | Available Moisture (AW)=Water Holding Capacity=Fc-PWP |  |  | AW% by volume =Asg x % by weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | % by volume cm/m | % by volume cm/m | % by volume cm/m | % by volume inch/foot |  | mm/m | inch/foot |
| Sandy | 15 | 7 | 8 | 1.00 |  | 82.5 | 0.99 |
|  | (10-20) Range | (3-10) | (7-10) | 0.8-1.2 |  |  |  |
| loam | 21 | 9 | 12 | 1.40 |  | 120.0 | 1.44 |
|  | (15-27) | (6-12) | (9-15) | 1.1-1.80 |  |  |  |
| Loam | 31 | 14 | 17 | 2.00 |  | 168.0 | 2.02 |
|  | (25-36) | (11-17) | (14-19) | 1.7-2.3 |  |  |  |
| Clay loam | 36 | 18 | 19 | 2.30 |  | 189.0 | 2.27 |
|  | (31-42) | (15-20) | (17-22) | 2.0-2.6 |  |  |  |
| Silty clay | 40 | 20 | 21 | 2.5 |  | 208.0 | 2.50 |
|  | (35-46) | (17-22) | (18-23) | 2.2-2.8 |  |  |  |
| Clay | 44 | 21 | 23 | 2.7 |  | 225.0 | 2.70 |
|  | (39-49) | (19-24) | (20-25) | 2.4-3.0 |  |  |  |
| Source: Principles of Farm Irrigation System Design by Larry G. James 1988 |  |  |  |  |  |  |  |

Available Soil Water Holding Capacity (AW)

| Soil Texture | Available Moisture (AW)/WHC mm/m Range Average |  | Inch/Foot |
| --- | --- | --- | --- |
| 1. Very coarse texture-very coarse sand | 33-62 | 42 | 0.50 |
| 2. Coarse texture-coarse, fine and loamy sands | 62-104 | 83 | 1.00 |
| 3.Moderately coarse textured-sandy loams | 104-145 | 125 | 1.50 |
| 4.Medium texture-very fine sandy loam, loam, silt loam | 125-192 | 167 | 2.00 |
| 5.Moderately fine texure-clay loams, silty-clay loam, sandy clay loam | 145-208 | 183 | 2.20 |
| 6.Fine texture-sandy clay, silty clay, clay | 133-208 | 192 | 2.30 |
| 7.Peat, muck | 167-250 | 208 | 2.50 |
| Note: 1 mm/m = 0.012 inch/foot |  |  |  |
| Source: Sprinkle and Trickle Irrigation by Jack Keller |  |  |  |

Average Rooting Depth (Rd)

| CATEGORY | CROP | mm | Foot |
| --- | --- | --- | --- |
| Shallow root depth-<500 mm | Cabbage | 400 | 2 |
|  | Cauliflower | 400 | 2 |
|  | Garlic | 400 | 2 |
|  | Ginger | 500 | 2 |
|  | Onion | 400 | 2 |
|  | Rice | 300 | 1 |
|  | Spinach | 400 | 2 |
|  | Turmeric | 500 | 2 |
| Medium root Depth -500 to 1000 mm | Banana | 700 | 3 |
|  | Bitter gourd | 600 | 2 |
|  | Carrot | 500 | 2 |
|  | Chickpea | 600 | 2 |
|  | Chilli | 700 | 3 |
|  | Coriander | 600 | 2 |
|  | Groundnut | 600 | 2 |
|  | Lentil | 600 | 2 |
|  | Mustard | 600 | 2 |
|  | Papaya | 600 | 2 |
|  | Pointed gourd | 900 | 4 |
|  | Potato | 500 | 2 |
|  | Radish | 500 | 2 |
|  | Sesbania | 600 | 2 |
|  | Soybean | 700 | 3 |
|  | Tomato | 1000 | 4 |
|  | Turnip | 500 | 2 |

Average Rooting Depth (Rd)

| CATEGORY | CROP | mm | Foot |
| --- | --- | --- | --- |
| Deep root -1000 mm and above | Bottle gourd | 1200 | 5 |
|  | Citrus | 1100 | 4 |
|  | Cotton | 1200 | 5 |
|  | Cucumber | 1200 | 5 |
|  | Egg plant (Brinjal) | 1000 | 4 |
|  | Fruit Trees | 1500 | 6 |
|  | Grapes | 1200 | 5 |
|  | Lady's finger (Okra) | 1000 | 4 |
|  | Maize | 1300 | 5 |
|  | Pumpkin | 1200 | 5 |
|  | Sugarcane | 1500 | 6 |
|  | Sweet potato | 1200 | 5 |
|  | Watermelon | 1200 | 5 |
|  | Wheat | 1200 | 5 |

Maximum Allowable Depletion (MAD)%

| CROP TYPE | CROP | MAD (%) |
| --- | --- | --- |
| CEREALS | Wheat | 50 |
|  | Maize | 50 |
|  | Rice | 80-100 |
| OIL SEEDS | Mustard | 50 |
|  | Groundnut | 50 |
|  | Soybean | 50 |
| GOURDS | Pumpkin | 50 |
|  | Watermelon | 50 |
|  | Bittle gourd | 50 |
|  | Cucumber | 50 |
|  | Bottle gourd | 50 |
| ROOTAND TUBER CROPS | Radish | 50 |
|  | Potato | 70 |
|  | Sweet potato | 50 |
|  | Carrot | 50 |
| VEGETABLES | Tomato | 65 |
|  | Egg plant | 65 |
|  | Cauliflower | 65 |
|  | Spinach | 65 |
| SPICES | Chilli | 60 |
|  | Coriander | 65 |
|  | Garlic | 70 |
|  | Garlic | 50 |
|  | Onion | 70 |
|  | Turmeric | 50 |
| FRUIT | Banana | 65 |
|  | Papaya | 50 |
| SUGAR CROPS | Sugarcane | 70 50 |

Flood Irrigation

Sprinkler Irrigation

Drip Irrigation

HOW MUCH DEPTH OF WATER T BE APPLIED (MM) FOR FULL IRRIGATION

Maximum Irrigation Depth to be applied for FULL IRRIGATION

Id = TAW x MAD(%)

Id 	=	Maximum net depth of water to be applied per irrigation, mm (in.)

This is the same as RAW

AW 	= 	(FC) – (PWP) or figures of AW are available

TAW	=	AW x Rd

MAD 	= 	Management Allowable Deficit of the soil

For example if Aw= 125 mm/m, Rd=1.2 m, MAD= 50% then

TAW = 125 x 1.2 = 150 mm

Id = 150 x 50/100 = 75 mm= RAW

Maximum Irrigation Interval

Ii = ____________Id________________

Peak Daily Crop Water Requirement

Where

Ii = Irrigation interval or frequency (days)

Id = Net depth of water application per irrigation to meet Water Requirement

ETc = Daily crop water requirement

For example if Peak ETc is 9.77 mm/day, then irrigation interval = 75/9.77 = about 8 days

Exercise-1

For the following data of ETo, given in mm/day.

Jan Feb Mar Apr May Jun Jul Aug Sep Oct Nov Dec

4.3  4.1  4.2  3.9  3.3  2.9  3.2 4.2  5.5  6.2  4.9  4.3

Kc ini = 0.59, Kc mid = 1.20, Kc end = 0.35

Estimate the crop evapotranspiration for a maize crop planted on 15 October on the project site. The soils are heavy textured.

The first step is to establish the length of the growth stages:

- Based on local experience, the duration of the initial stage is expected to be 20 days for a maize crop planted on 15 October, because of the favorable weather conditions. The development stage, also from local experience, will be 45 days and the mid-season stage will be 50 days. The late season stage is expected to last 39 days.

- Therefore the crop will finish around the 15th of March.

- The Kc curve for maize can now be drawn, for initial planning purposes, where Kc ini , Kc mid, and Kc end are 0.59, 1.2 and 0.35 respectively, and the four lengths of growth stages are 20, 45, 50 and 39 days.

- Using the ETo figures and the Kc values, the crop evapotranspiration ETc for maize can be calculated. For a decade within one month, the daily ETo within the month is multiplied with the corresponding Kc to derive the ETc.

- For the first full decade in October ETc = 6.2 x 0.59 = 3.7 mm/day.

- For a decade that falls in two months, the weighted average of daily ETo from each month is multiplied by the corresponding Kc. For example, the ETc of decade 2 (end of October and early November) is calculated as follows:

- ETo in October = 6.2 mm/day and ETo in November = 4.9 mm/day.

- The planting date being 15 October means that decade 2 has 5 days in October and 5 days in November. The weighted ETo would be: (5/10) x 6.2+ (5/10) x 4.9 = 5.6 mm/day. This proportional ETo would then be multiplied by the corresponding Kc.

Exercise-2

Maiz has to be grown on July 21st  at Agriculture Farm IUB, District Bahawalpur.

Estimate the crop water requirement (mm/day) for whole of the growing period (at 10 day interval).