Chapter # 11

Evaluation of potency of disinfectants, anti-infective and antibiotics

Submitted to: Prof. Dr. Naveed Akhtar

Class: M.Phil. (Pharmaceutics)

Subject: Microbiology

Department of Pharmacy

The Islamia University of Bahawalpur

Terminologies:

Disinfectants:

Chemical agent that kills the vegetative forms of infectious microorganisms but not necessarily kills their spores. The term normally refers to substance used on inanimate objects

Antibiotic:

A drug used to treat bacterial infections.

Anti-Infective:

Anti-infective agents are drugs capable of acting against infection by inhibiting the spread or by killing the infectious agents.

# Disinfectant:

PROPERTIES OF AN IDEAL DISINFECTANT:

- Must be fast acting

- Effective against most infectious agents

- Must be stable to light and heat etc

- Easy to prepare

- Inexpensive and easy to obtain

- Not have an unpleasant odour

Major classes of disinfectants:

- Phenol  and related compounds (lysol)

- Alcohols (ethanol)

- Halogens (iodine, chlorine)

- Heavy metals and their compounds (silver, mercury)

- Detergents

- Dyes

- Quaternary ammonium compounds

- Aldehydes (formaldehyde)

Evaluation of Anti-microbial agents and Disinfectants Techniques and methods used for evaluation of Disinfectants:

1. Tube dilution and agar plate method.

2. Cup plate method or Cylinder plate method.

3. Ditch- Plate method.

4. Gradient plate technique.

5. Phenol coefficient method

METHOD FOR EVALUATION DISINFECTANTS:

  - dilution technique

  - Phenol coefficient technique

Test Organism: Salmonella typhi, staphylococcus aureus

Standard disinfectant: Phenol

PHENOL COEFFICIENT TEST:

It may be defined as “the killing power of a disinfectant compared with that of phenol.

Procedure:

- A series of tubes is prepared, each tube containing 5 ml amount of a different dilution of disinfectant to be tested.

- A second series of tubes containing various dilutions of phenol is prepared.

- The tubes of both series are inoculated with 0.5 ml from a 24 hours old broth culture of test bacterium.

- At intervals of 5, 10 and 15 minutes a sample is removed from each tube and inoculate into a tube of sterile growth medium

- The tubes of inoculated medium are incubated for 24-48 hrs. At 370C and examined to see whether the test organism has grown.

Calculation of phenol coefficient:

The greatest dilution of test disinfectant that kills the test organism in 10 min but not in 5 min is divided by the greatest dilution of phenol showing the same result. The number obtained is the phenol coefficient for that particular disinfectant.

Interpretation of results:

  - Phenol Coefficient (PC) for phenol is 1

  - A PC value higher than 1 indicates that the chemical agent is more effective than phenol

  - A PC value lower than 1 indicates that the chemical agent is less effective than phenol

Dilution method:

- A 48 hour culture of an individual species of bacteria is dried onto a number of small, cylindrical, stainless steel test surfaces called penicylinders (referred to as "carriers").

- The carriers are dried.

- Each dry, contaminated test carrier is transferred, individually, to a test tube filled with 10 mL of disinfectant,

- The treated test surfaces are incubated in the neutralizing growth medium for 48 hours.

- After incubation, the number of tubes showing growth of the target microorganism is recorded.

# ANTI_INFECTIVE AGENTS/ ANTIBIOTIC AGENTS:

Anti-infective agents are drugs capable of acting against infection by inhibiting the spread or by killing the infectious agents. It includes

- Antibacterial

- Antitubercular

- Antiviral

- Antifungal

- Antiprotozoal

EVALUATION METHOD Of Anti biotic /Anti-infective:

- TUBE  DILUTION TEST

- AGAR PLATE METHOD

TUBE DILUTION TEST:

- Dilution methods are used to determine the minimum inhibitory concentrations (MICs) of antimicrobial agents and are the reference methods for antimicrobial susceptibility testing.

- The MICis defined as the lowest concentration of an antimicrobial agent that inhibits the growth of a microorganism.

Procedure:

- Preparation of test microorganism :

A pure culture of a single microorganism is grown in Mueller-Hinton broth or other broth as appropriate. The more standard the microbial culture, the more reproducible the test results.

- Preparation of antibiotic dilution:

The antimicrobial agent is diluted a number of times, usually at a ratio 1:1 through a sterile diluent (typically using Mueller-Hinton broth)

- Preparation of inoculum

Prepare the inoculum by making broth suspension from pure culture. Adjust the suspension to achieve a turbidity equivalent to a 0.5 McFarland turbidity standard. Dilute the adjusted inoculum suspension in broth

- Inoculation

Within 15 minutes after the inoculum has been standardized as described above, add volume of inoculum equal to the volume of the diluted antimicrobial agent to each dilution vessel.

- Incubation

Incubate the inoculated tubes at 35 ± 2 ºC for 16 to 20 hours in an ambient air incubator. The more consistent the incubation period, the more reproducible the test results.

- Interpretation

Examine tubes for visible signs of bacterial growth. The lowest concentration of antibiotic that prevented growth represented the minimal inhibitory concentration (MIC).

ADVANTAGE:

- MIC is important in diagnostic laboratories to confirm resistance of microorganisms to an antimicrobial agent.

DISADVANTAGES

- manual task of preparing the antibiotic solutions for each test,

- possibility of errors in preparation of the antimicrobials solutions,

- Relatively large amount of reagents and space required for each test.

AGAR PLATE METHOD:

This method is used in many clinical microbiology laboratories for routine antimicrobial susceptibility testing.

Procedure:

- The test is performed by applying a bacterial inoculum of approximately 1–2×108CFU/mL to the surface of a large (150 mm diameter) Mueller-Hinton agar plate.

Then, filter paper discs (about 6 mm in diameter), containing the test compound at a desired concentration, are placed on the agar surface

- Plates are incubated for 16–24 h at 35°C prior to determination of results.

- Generally, antimicrobial agent diffuses into the agar and inhibits germination and growth of the test microorganism and then the diameters of inhibition growth zones are measured

ADVANTAGES:

- Simplicity

- Low cost

- The ability to test enormous numbers of microorganisms and antimicrobial agents.

- Easy to interpret results.

MCQs

Choose the appropriate answer

- Phenol co-efficient method also known as:

- Rideal walker test

- Dilution test

- Tube test

- Phenol test

2- In phenol co-efficient test each tube containing ----- amount of a different dilution of disinfectant to be tested:

A) 2ml

B) 5ml

C) 10ml

D) 15ml

3-A PC value ----- indicates that the chemical agent is more effective than phenol:

- Less than 1

- More than 1

- Equal to 1

- none

4- ------ are used to determine the minimum inhibitory concentrations (MICs) of antimicrobial agents:

- Tube dilution  test

- Phenol test

- Cup plate method

- All of these

5- The antimicrobial agent is diluted a number of times, usually at a ratio ----- through sterile diluents:

- 1:5

- 1:4

- 1:3

- 1:1

6- Incubate the inoculated tubes at ------ for 16 to 20 hours in an ambient air incubator:

- 37 ± 2 ºC

- 35 ± 2 ºC

- 32 ± 2 ºC

- 39± 2 ºC

7- Filter paper discs   (------in diameter), containing the test compound at a desired concentration, are placed on the agar surface:

- 9mm

- 8mm

- 6mm

- 7mm

8-phenol and their related compounds are:

- ETHANOL

- LYSOL

- IODINE

- CHLORINE

9- Phenol coefficient test is suitable for testing:

- Disinfectants

- Antibiotics

- Antiseptics

- All of these

10-The disinfectant to be tested is compared with phenol on a standard microbe:

- Salmonella typhi

- Staphylococcus aureus

- Both A and B

- None of them

TRUE AND FALSE

- Disinfectants are the chemical agent that kills the vegetative forms of infectious microorganisms but not necessarily kills their spores.   T

- Phenol coefficient test is suitable for testing antibiotics. F

- The disinfectant to be tested is compared with phenol on a standard microbe is salmonella typhi. T

- Phenol Coefficient (PC) for phenol is greater than 1. F

- In dilution method, a 48 hour culture of an individual species of bacteria is dried onto a number of small, cylindrical, stainless steel test surfaces called Petri dishes. F

- Dilution methods are used to determine the minimum inhibitory concentrations (MICs) of antimicrobial agents. T

- The MICis defined as the lowest concentration of disinfectant that inhibits the growth of a microorganism.  F

- Incubate the inoculated tubes at 35 ± 2 ºC for 16 to 20 hours in an ambient air incubator. T

- The highest concentration of antibiotic that prevented growth represented the minimal inhibitory concentration (MIC). F

- Antimicrobial agent diffuses into the agar and inhibits germination and growth of the test microorganism and then the diameters of inhibition growth zones are measured. T