Chapter # 08
Sterilization of biological products (proteins, vaccines, toxoids and hormones)

Subject:

Pharmaceutical Microbiology

Subject Supervisor:

Prof. Dr. Naveed Akhter

Department of Pharmacy

The Islamia University of Bahawalpur

INTRODUCTION

Sterilization:

- The inactivation of all self-propagating biological entities (e.g. bacteria, viruses, prions) associated with the materials or areas under consideration.

Sterile:

- The term sterile means complete freedom from all viable microorganisms.

Biological Products:

A biological product is defined as “

- a virus,

- toxoid,

- vaccine,

- blood,

- blood component or derivative,

Applicable to the prevention, treatment or cure of a disease or condition of human beings”.

Most common biological products are:

- Proteins

- Hormones

- Vaccines

- Toxoids

STERILISATION OF VACCINES:

VACCINE:

- Biological preparation

- Improves immunity

- Contains agent, which resembles to disease inducing microorganism (bacteria,virus,toxin etc.)

- Activates body immune system

- TYPES OF VACCINE :

Sterilization of inactivated vaccine

Procedure:

- Inactivation of live-virus by psoralen derivatives.

- In non-oxidizing gas atmosphere,(degradation of antigenic characteristics resulting from U.V irradiation is prevented.)

- U.V irradiation(300-400 micrometers) for 2-20hrs.

- Dialysis to remove furocumarin compounds

- Add adjuncts to keep sterile.

Sterilization of live-attenuated vaccine

During manufacture:

- Treatment of air.

- Establish pressure gradient to control direction of air movement.

- Special workstations.

- Laminar flow cabinets.

During filling:

- Use of :

  - Laminar flow work station

  - Disinfecting agents

  - Sterilized clothing

  - Germicidal lamps

How to keep sterile?

Antibiotics added in some vaccines:

- Help prevent bacterial contamination during use.

- Decrease bacterial growth in embryonic eggs during processing.

Formaldehyde:

- Inactivate vaccine.

- Detoxify bacterial toxin.

Sterilization of toxoid

- Diphtheria toxoid is entrapped within liposomes.

- Various methods are used to sterilize diphtheria toxoid

- Lyophilization:

- Also called freeze drying

2. Cryoprotectants:

- Mechanism of stabilization by saccharides (prevent aggregation of vesicles.)

3. Gamma radiations:

- In dry state ( freeze-dried)

Mechanism:

- The major target for radiation is microbial DNA.

- Gamma rays cause ionization.

Sterilization of proteins

What is Protein?

- Any of a class of nitrogenous organic compounds, which have large molecules composed of one or more long chains of amino acids.

Proteins Storage:

- Purified proteins often need to be stored for an extended period while retaining their activity.

- Proteins are best stored at ≤ 4°C.

- For short-termstorage (1 day to a few weeks), many proteins may be stored in simple buffers at 4°C.

- For Long Term Storage for 1 month to 1 year, choose protein in liquid nitrogen for storage in clean plastic containers under liquid nitrogen.

- Frozen at -20°C or -80°Cis the more common form of cold protein storage. Because freeze-thaw cycles decrease protein stability.

Method for sterilizing recombinant human serum albumin pharmaceutical preparation

Sterilization of rHSA:

- A recombinant human serum albumin (rHSA) pharmaceutical preparation is sterilized by subjecting a pharmaceutical preparation of rHSA packed in a container in an administration unit to heat treatment at 50° to 80° C for 30 minutes or more.

Sterilization for viral contamination:

- The danger of viral contamination becomes extremely high when the preparations produced using blood material collected from a virus infected blood donor.

To inactivate Viruses several processes have been proposed,

- A process in which a compound such as β-propiolactoneis used jointly with UV rays radiation.

- A process in which heat treatmentis employed.

Heat treatment:

- carried out by low temperature disinfection (pasteurization)

- Employed in the final step.

- Hepatitis virus can be inactivated by 10 hours of heat treatment at 60° C. (pasteurization)

- The thermal stability of albumin during heat treatment can be ensured by the addition of a stabilizing agent, such as sodium acetyl tryptophan.

- The possibility of causing microbial contamination during production of the pharmaceutical preparation is also extremely low, because the preparation is subjected to sterile filtration at the final step and then dispensed and packed aseptically.

Sterilization after packing:

- The present invention relates to a method for sterilizing a rHSA pharmaceutical preparation which is packed in a container is subjected to heat treatment at 50° to 80° Cfor 30 minutes or more.

Radiation sterilization:

The method comprises the steps

- forming a mixture that comprises protein

- Cooling the mixture to a temperature sufficient to freeze.

Maintenance of sterility:

- Sterilizing agents can be added  e.g. Methiolate (A sterilizing agent for albumin)

- Packing should be aseptic.

STERILIZATION OF HORMONES

Definition:

- A hormone is any member of a class of signalling molecules produced by glands in multicellular organisms that are transported by the circulatory system to target distant organs to regulate physiology and behaviour.

Insulin

- Insulin is a polypeptide hormone that regulates carbohydrate metabolism

- Insulin is produced in the islets of Langerhans in the pancreas

Insulin sterilization during preparation:

- Instruments that have undergone sterilization should be maintained in such condition by contaminant.

- Aseptic technique is the act of maintaining sterility.

- Gamma radiations and membrane filtration methods are used to sterilize the insulin during preparation.

How it works:

- The object to be sterilized are put on conveyer, which transport them to the vicinity of a strong gamma radiations source such as cobalt-60

- The ionizing radiation causes excitations, ionizations and where water is present, free radical formation.

- Free radicals are powerful oxidizing and reducing agent, capable of damaging essential molecules in living cells.

Sterilization of insulin before filling

- Membrane filtration method is used to sterilize the insulin before filling

- Membrane filtration method

- Filter should have nominal pore size of not more than 0.22 micrometre.

- These filters can remove the bacteria and moulds but not virus and yeast.

- To remove viruses and yeast filter with pore size of 20-50nm is used.

How to maintain sterility after packaging

- When product is packed they will remain sterile, there is no need of sterilization.

- After packaging gamma rays can be passed through the containers in which insulin is packed

- Sterility of insulin product is maintained by storing at refrigerator and room temperature

- Never store insulin in the freezer or direct sunlight

Sterility testing

Two methods are used for sterility testing of insulin:

- Membrane filtration method

- Direct transfer

Media used:

- Fluid thioglycolate medium

- Soya bean casein medium

- Sauboured media

MCQs

1. Which of the following is an example of live vaccine?

A. pertusis

B. mumps

C. cholera

D. rabies

2. Triple toxoid vaccine gives protection against

A. Diphtheria, tetanus and rabies

B. Tetanus, whooping cough, Tuberculosis

C. Whooping cough, tetanus and Diphtheria

D. Whooping cough, cancer and T.B.

3. Diphtheria toxoid is prepared by using

A. Aldehyde

B. Formalin

C. Phenols

B. None of these

4. A biological product contain:

A. Hormones

B. Proteins

C. Vaccines

D. All of these

5. During Manufacturing, sterilization of live attenuated vaccine is done by:

A. Sterilized clothing

B. Disinfecting agents

C. Germicidal lamps

D. Treatment of air.

6. Diphtheria toxoid is entrapped within:

A. Niosomes

B. Proteins

C. Liposomes

D. None of these

7. Proteins are best stored at temperature:

A. >4°C

B. ≤ 4°C

C. <4°C

D. None of these

8. The genetic material in HIV is:

A. ds DNA

B. ss DNA

C. s RNA

D. None of these

9. For which purpose “Adjuncts” are added in the process of sterilization for inactivated vaccine.

A. To keep sterile

B. To keep disinfectant

C. To keep stable

D. All of these

10. Methods are used to sterilize the insulin during preparation:

A. Gamma radiations

B.Membrane filtration

C. Both A & B

D. None of these

True/False

1.  Vaccines Activates body immune system (TRUE)

2. In Inactivated vaccine sterilization U.V radiation (300-400 micrometers) for 2hrs (FALSE).

3. Formaldehyde detoxify the Viral Toxins (FALSE).

4. Hepatitis virus can be inactivated by 10 hours of heat treatment at 60° C. (pasteurization) (TRUE).

5. A hormone is any member of a class of signaling molecules produced by glands in multicellular organisms that are transported by the circulatory system to target distant organs to regulate physiology and behavior.(TRUE).

6. For insulin sterilization the object to be sterilized are put on conveyer, which transport them to the vicinity of a strong gamma radiations source such as Cobalt-30 (FALSE).

7. To remove viruses and yeast filter with pore size of 20-50nm is used (TRUE).\

8.  After packaging UV rays can be passed through the containers in which insulin is packed (FALSE).

9. Insulin always store in freezer or direct sunlight (FALSE).

10. Membrane filtration method is used to sterilize the insulin before filling (TRUE).