Chapter # 09

Calibration of instruments used for sterilization (Autoclave, laminar-flow hood- hot air oven etc.). Validation of sterilization procedures

Submitted to:

Prof. Dr. Naveed Akhtar

The Islamia University of Bahawalpur

Calibration & Validation

Calibration is the comparison of measurement values delivered by a device under test with those of a calibration standard of known accuracy. Such a standard could be another measurement device of known accuracy, a device generating the quantity to be measured.

Establishing documented evidence which provides high degree of assurance that specific process will be consistently produce a product meeting its predetermined specifications &  quality attributes.

Sterilization

Any physical or chemical process which destroys all life forms, with special  regard to microorganisms (including bacteria and sporogenous forms), and  inactivates viruses. Therefore, the terms "sterile" and "sterilization", in a strictly biological sense, describe the absence or destruction of all viable microorganisms. In other words, they are absolute terms: an object or system is either "sterile" or "not sterile".

Why Are Autoclaves Essential?

- Easiest way to sterilize large volumes of heat tolerant materials.

  - More effective than dry heat (lower temperature /shorter  time

  - Not as messy as chemicals and more reliable

  - No need for radiation shielding etc.

- Once validated, simple indicators used to tell autoclaved and  non autoclaved material apart – the temp/time/pressure trace  is used to confirm sterilization occurred.

- Can deliver > 1012 sterility assurance

Definitions: D-Value, Z-Value and Fo

What is the D value?

  - Refers to decimal reduction time - The time required at a certain temperature to kill 90% (reduce population by log 1) of the organisms being studied. Thus after an organism is reduced by 1 D, only 10% of the original organisms remain. Dependent on microbe and initial numbers.  Eg D value of 1.5 means it takes 1.5minutes to reduce 1 log (to 10%) @121oC. A D value of 2.0 means more resistant while a Dvalue of 1min means less resistant.

What is a Z value?

  - Refers to the temperature change required to produce a 1-logreduction in D value.

What is F0?

The number of minutes to kill a specified number of microbes with specified Z value at specified temperature.

Often confused with the time the chamber is held at elevated temperature and pressure and in practice is the same thing.

Calibration of Sterilisation Processes

- Biological measurements

  - Required to demonstrate that  sterilisation process was  effective

  - Physical measurements

  - Time, temperature, pressure, vacuum.

  - Required to calculate sterility assurance levels (SAL)

- Chemical measurements

  - Autoclave tape or other indicators such as Bowie Dick

How Does an Autoclave Sterilize

- Steam held at elevated  temperature and pressure for  time is used to transfer moist  heat.

- The steam condenses on a surface and releases energy

- The energy splits open the cell wall.

- Heat acts to denature proteins, effectively killing all cells present.

- Effectiveness is reliant on saturated steam condensing

LAMINAR FLOW HOODS & BIOLOGICAL SAFETY CABINETS

What are HEPA (High Efficiency Particulate Air) filters?

•   They consist of a thin pleated sheet of boron silicate microfibers with aluminum separators

•They are particulate filters, which retain airborne particles, and microorganisms (gases pass freely through)

How are HEPA filters rated?

•   HEPA filters are rated on their ability to retain particles of around 0.3 microns in size

•   tested with PAO (poly alpha olefin) which produces particles of 0.26 microns in size

•   Most aerosol droplets are greater than 0.26 microns

Laminar Flow Hoods

- Provide product protection only and must not be used when working with any form of biohazard or chemical hazard

- Any potentially infectious aerosol that is created will lead to exposure of the operator and the environment

- Horizontal-flow clean-air bench used for cell cultures can expose the researcher to aerosols of allergenic or infectious materials.

- Vertical-flow clean-air bench also blows air out into the room

Biological Safety Cabinets

- Provide personnel and environment protection and usually product protection

- Infectious agents must be used in a biological safety cabinet NOT a laminar flow hood.

Class I Biological Safety Cabinet:

- A ventilated cabinet, which provides personnel and environmental protection only

- Airflow is directed away from the researcher, but is not HEPA filtered, therefore there is no product protection

- similar to a fume hood with a HEPA filter on the exhaust system to protect against the release of biohazards.

Class II Biological Safety Cabinet:

- Provides personnel, product and environmental protection

- There are supply air and exhaust air HEPA filters

- Two general types: IIA cabinets have a minimum inward airflow of 75 fpm and recirculates

- 70% of the air; IIB cabinets have a minimum inward air flow of 100 fpm and exhaust either

- 70% (type B1) or 100% (type B2)

Class III Biological Safety Cabinet:

- These cabinets provide personnel, product and environmental protection

- They are hermetically sealed and all procedures are conducted through arm-length rubber gloves

•   used in high level (Level 4) containment labs

•   There are two HEPA filters on the exhaust system

MCQs

1.THE TECHNIQUE USED TO AVOID ALL MICROORGANISMS IS ACCOMPLISHED BY

A. STERILIZATION

B. DISINFECTION

C. SURGICAL STERILIZATION

D. DISINFECTION STERILIZATION

2. IN AUTOCLAVE, THE PRINCIPLE INVOLVED IS

A. DRY HEAT

B. MOIST HEAT

C. STEAM UNDER PRESSURE

D. BOTH B AND C

3. AT THE TEMPERATURE 160OC FOR ONE HOUR, COMPLETE STERILIZATION OCCURS IN

A. AUTOCLAVE

B. HOT AIR OVEN

C. LAMINAR FLOW

D. INCUBATOR

4. AUTOCLAVING IS CARRIED AT

A. DRY HEAT

B. ATMOSPHERIC PRESSURE

C. 120OC

D. ALL OF THESE

5.WHICH OF THE FOLLOWING METHOD OF STERILIZATION HAS NO EFFECT ON SPORES?

A. DRYING

B. HOT AIR OVEN

C. AUTOCLAVE

D. NONE OF THESE

6.  WHAT ARE LAMINAR FLOW HOODS WORK SURFACES MADE OF?

A. FORMICA

B. STAINLESS STEEL

C. BOTH A & B

D. NONE OF THESE

7. WHAT SIZE PARTICLES CAN HEPA FILTER REMOVE?

A. 0.3 MICRONS

B. 0.5 MICRONS

C. 0.2 MICRONS

D. NONE OF THESE

8. HEPA FILTERS CAN HAVE A LIFESPAN OF

A. 1-5 YEARS

B. 2-6 YEARS

C. 1-2 YEARS

D. 1-3 YEARS

9. AVERAGE Z VALUE OF BATIERIA “CLOSTRIDIUM BOTULINUM” IS:

A. 10

B. 20

C. 07

D. 13

10. ACCURACY OF THERMOCOUPLES SHOULD BE

A. ± 0.60C

B. ± 0.50C

C. ± 0.70C

D. ± 0.80C

True/False

1. Air is taken in and processed through pre-filter to remove contaminants. Air then blows from back of the hood through HEPA filter and across the work surface area outwards towards the technician.(FALSE).

2. Lysozyme is an enzyme that kills gram-positive bacteria (TRUE).

3. A bacterial spore consists of a hard, thick-walled capsule that can resist adverse conditions(TRUE).

4. The purpose of sanitization is to remove all microorganisms and spores from a contaminated article (FALSE).

5. According to OSHA, gloves do not need to be worn during the sanitization process (FALSE).

6.Autoclave tape indicates whether an autoclaved item is sterile (TRUE).

7.The wrapper used to autoclave articles should prevent contaminants from getting in during handling and storage (TRUE).

8. Class II biological safety cabinets, either 70% (B1) or 100% (B2) of the airflow is exhausted out of the cabinet (TRUE).

9. Class III biological safety cabinets, also known as glove boxes or barrier isolators (TRUE).

10.Laminar flow hoods fall in to four basic categories(FALSE).