The Islamia University of Bahawalpur

University College of Engineering and Technology

Department of Telecommunication Engineering

| Final Term Exam. Sample Paper | 4thSemester | Max. Marks: 50, Time : 120 Min |
| --- | --- | --- |
| Course: ES-221: Electronic Circuit Design |  | Instructor: Syed Abdul Rehman Rizvi |

Q1. (CLO 1)			                        		 	       [10 Marks]

- Discuss the configuration in which the gate terminal is grounded and the input signal is typically applied to the source terminal and the output signal is obtained at the drain terminal.

- Determine the following for the common-gate configuration of Fig given below

a. VGSQ      b. IDQ         c. V D      d. V G      e. V S     f. V DS

Q2. (CLO 1)

[10 Marks]

- Explain AC and DC Bias Operation of SERIES-FED Class A Amplifier. Also describe its Power consideration and maximum efficiency.

- Calculate the input power, output power, and efficiency of the amplifier circuit in Fig. given below for an input voltage that results in a base current of 10 mA peak.

Q3. (CLO 3)									[10 Marks]

- Draw an FET amplifier stage with voltage-series feedback. A part of the output signal (Vo) is obtained using a feedback network of resistors R 1 and R 2 . The feedback voltage Vf is connected in series with the source signal Vs, their difference being the input signal Vi . Find the gain without feedback and with negative feedback.

- Calculate the gain without & with feedback for the FET amplifier stage with voltage-series feedback and the following circuit values: R1 = 80 k Ohm, R2 = 20 k Ohm, Ro = 10 k Ohm,     RD = 10 k Ohm, and gm = 4000 mS.

Q4. (CLO4)									[10Marks]

Discuss the Block representation of push–pull operation. Illustrate the Input (DC) Power, Output (AC) Power and Efficiency of Class B Amplifier.

Q5. (CLO5)									[10 Marks]

What are oscillators? Illustrate Practical RC phase-shift oscillator circuits using BJT. Derive the frequency at which the phase shift is exactly 180°.