The Islamia University of Bahawalpur

University College of Engineering and Technology

Department of Telecommunication Engineering

| B. Sc. Telecommunication Engineering |  | Time Allowed: 75 Mins |
| --- | --- | --- |
| Mid Examinations, Sample paper | 6Th Semester | Max. Marks: 30 |
| Course: CS-343: Computer Communication Networks |  | Instructor: Engr. M.N.Abbasi |

Question No.1:  Short Questions					          [2x6= 12 Marks] CLO: 01

- Consider an e-commerce site that wants to keep a purchase record of its customers. Briefly explain how this can be done with cookies.

- Differentiate between SMTP and FTP

- Briefly describe the difference between the propagation and transmission delay.

- Differentiate between the Client & server and Peer to Peer architecture.

- Differentiate between the  versions of RDT 2.1 and RDT 2.2?

- Write the names of data unit at each layer, that passes from Application layer down to the data link layer.

Question No.2								[3+4=7 Marks] CLO:01

(A): Provide the hierarchical database distribution of DNS servers. Let a client clicks on the link to access www.amazon.com. Show the access protocol for DNS servers.

(B): Justify how Web Cache reduces the traffic intensity.

Question No.3 								        [6 Marks] CLO:02

Perform UDP Checksum to determine the error in the data assume that the channel has introduced the error in the bits while transmission. Consider the following 3 16-bit words.

0110011001100000

0101010101010101

1000111100001100

Question No.4.                                                                                                       [5 Marks] CLO:02

Suppose two hosts, A and B, are separated by 10,000 kilometers and are connected by a direct link of R = 4 Mbps. Suppose the propagation speed over the link is 2.5 x108 meters/sec.

a. Calculate the bandwidth-delay product.

b. Consider sending a file of 800,000 bits from Host A to Host B. Suppose

the file is sent continuously as one large message. Determine the maximum

number of bits that will be in the link at any given time?

c. Determine is the width (in meters) of a bit in the link?

****BEST OF LUCK****