The Islamia University of Bahawalpur

University College of Engineering and Technology

Department of Telecommunication Engineering

| B. Sc. Telecommunication Engineering | Time Allowed: 105 minutes |
| --- | --- |
| Final-Term Examination, Sample Paper | Max. Marks: 40 |
| Course: Computer Communication Network | Instructor: Engr.M. Nawaz Abbasi |
| Note: Attempt all questions. |  |

Q. No.1.  							 Marks [02+ 02 + 01+4] CLO 1

- Distinguish between the RTP and RTCP.

- Differentiate between SIP & H.323

- Explain briefly Private IPs.

- Distinguish between the Go back N  and Selective Repeat operations .

.Q.2 Using Distance Vector routing algorithms, determine the shortest route from A to all other Nodes.                                                                                                       Marks [10] CLO 2

Q.3a. 										    Marks [10] CLO 3

You are the network administrator of the university containing Six different departments. Your task is to design the subnets with network address 12.0.0.0.The maximum No of hosts requirement to a department is 60.

- Infer the class of the network address

- Determine the number of subnets required.

- Determine the default subnet mask and the new customised subnet mask

- Determine  the decimal value range of each subnet starting from first IP address and broadcasting IP address of each subnet.

- Infer the last IP address and the second last IP address after the complete subnetting of the said IP address

Q.3 (b). 								            Marks [07] CLO 3

Consider the IP address 192.168.15.120/28 , Design the subnets of the said IP address  and infer that the IP address belongs to which  subnet ID, Broadcast ID of the subnet,  and subnet number.

Q4. 										Marks [06] CLO 2

For the data = 101010 and divisor = 1100 apply CRC at the sender side and detect any error at the receiver side. Now consider due to noise in the medium one data bit has changed, again perform CRC and detect error at the receiving side.

***BEST OF LUCK***

3

A

B

3

2

3

2

3

4

C

E

D