Classless Addressing or CIDR

## Classless Addressing-

- Classless Addressing is an improved IP Addressing system.

- It makes the allocation of IP Addresses more efficient.

- It replaces the older classful addressing system based on classes.

- It is also known as Classless Inter Domain Routing (CIDR).

## CIDR Block-

When a user asks for specific number of IP Addresses,

- CIDR dynamically assigns a block of IP Addresses based on certain rules.

- This block contains the required number of IP Addresses as demanded by the user.

- This block of IP Addresses is called as a CIDR block

## CIDR Notation-

CIDR IP Addresses look like-

a.b.c.d / n

- They end with a slash followed by a number called as IP network prefix.

- IP network prefix tells the number of bits used for the identification of network.

- Remaining bits are used for the identification of hosts in the network.

## Example-

An example of CIDR IP Address is-

182.0.1.2 / 28

It suggests-

- 28 bits are used for the identification of network.

- Remaining 4 bits are used for the identification of hosts in the network.

## Rules For Creating CIDR Block-

A CIDR block is created based on the following 3 rules-

## Rule-01:

- All the IP Addresses in the CIDR block must be contiguous.

## Rule-02:

- The size of the block must be presentable as power of 2.

- Size of the block is the total number of IP Addresses contained in the block.

- Size of any CIDR block will always be in the form 21, 22, 23, 24, 25 and so on.

## Rule-03:

- First IP Address of the block must be divisible by the size of the block.

Example

We have-

- Size of the block = Total number of IP Addresses = 24.

- To have 24 total number of IP Addresses, total 4 bits are required in the Host ID part.

- According to Rule-03, first IP Address must be divisible by size of the block.

- So, 100.1.2.32 must be divisible by 24.

- 100.1.2.32 = 100.1.2.00100000 is divisible by 24 since its 4 least significant bits are zero.

- So, Rule-03 is satisfied.