Congestion Control

The main focus of congestion control and quality of service is data traffic. In congestion

control we try to avoid traffic congestion.

Traffic Descriptor

Traffic descriptors are qualitative values that represent a data flow.

Average Data Rate

The average data rate is the number of bits sent during a period of time, divided by the

number of seconds in that period. We use the following equation:

Average data rate =  amount of data

Time

Peak Data Rate

The peak data rate defines the maximum data rate of the traffic.

Maximum Burst Size

The maximum burst size normally refers to the maximum length of time the traffic is generated at the peak rate.

Effective Bandwidth

The effective bandwidth is the bandwidth that the network needs to allocate for the

flow of traffic.

Traffic Profiles

For our purposes, a data flow can have one of the following traffic profiles: constant bit

rate, variable bit rate, and bursty.

Constant Bit Rate

A constant-bit-rate (CBR), or a fixed-rate, traffic model has a data rate that does not

change.

Variable Bit Rate

In the variable-bit-rate (VBR) category, the rate of the data flow changes in time, with the changes smooth instead of sudden and sharp.

Bursty

In the bursty data category, the data rate changes suddenly in a very short time. It may jump from zero

CONGESTION

Congestion in a network may occur if the load on the network-the number of packets sent to the network-is greater than the capacity of the network-the number of packets a network can handle.

Congestion control refers to the mechanisms and techniques to control the congestion and keep the load below the capacity.

Network Performance

Congestion control involves two factors that measure the performance of a network: delay

and throughput.

Delay Versus Load

Note that when the load is much less than the capacity of the network, the delay is at a minimum. This minimum delay is composed of propagation delay and processing delay, both of which are negligible. However, when the load reaches the network capacity, the delay increases sharply because we now need to add the waiting time in the queues (for all routers in the path) to the total delay.

Throughput Versus Load

We can define throughput in a network as the number of packets passing through the network in a unit of time. Notice that when the load is below the capacity of the network, the throughput increases proportionally with the load. We expect the throughput to remain constant after the load reaches the capacity, but instead the throughput declines sharply. The reason is the discarding of packets by the routers.

Congestion Control

Congestion control refers to techniques and mechanisms that can either prevent congestion, before it happens, or remove congestion, after it has happened. In general, we can divide congestion control mechanisms into two broad categories: open-loop congestion control (prevention) and closed-loop congestion control (removal).

Open-Loop Congestion Control

In open-loop congestion control, policies are applied to prevent congestion before it happens. In these mechanisms, congestion control is handled by either the source or the destination.

Retransmission Policy

Retransmission is sometimes unavoidable. If the sender feels that a sent packet is lost or corrupted, the packet needs to be retransmitted. Retransmission in general may increase congestion in the network. However, a good retransmission policy can prevent congestion. The retransmission policy and the retransmission timers must be designed to optimize efficiency and at the same time prevent congestion.

Window Policy

The type of window at the sender may also affect congestion. The Selective Repeat window is better than the Go-Back-N window for congestion control. In the Go-Back-N window, when the timer for a packet times out, several packets may be resent, although some may have arrived safe and sound at the receiver. This duplication may make the congestion worse. The Selective Repeat window, on the other hand, tries to send the specific packets that have been lost or corrupted.

Acknowledgment Policy

The acknowledgment policy imposed by the receiver may also affect congestion. If the receiver does not acknowledge every packet it receives, it may slow down the sender and help prevent congestion

Closed-Loop Congestion Control

Closed-loop congestion control mechanisms try to alleviate congestion after it happens. Several mechanisms have been used by different protocols.