Flow Control

Flow control is a technique that allows two stations working at different speeds to communicate with each other. It is a set of measures taken to regulate the amount of data that a sender sends so that a fast sender does not overwhelm a slow receiver. In data link layer, flow control restricts the number of frames the sender can send before it waits for an acknowledgment from the receiver.

## Stop and Wait Protocol-

Stop and Wait Protocol is the simplest flow control protocol.

It works under the following assumptions-

- Communication channel is perfect.

- No error occurs during transmission.

## Working-

- Sender sends a data packet to the receiver.

- Sender stops and waits for the acknowledgement for the sent packet from the receiver.

- Receiver receives and processes the data packet.

- Receiver sends an acknowledgement to the sender.

- After receiving the acknowledgement, sender sends the next data packet to the receiver.

## Limitations-

It is extremely inefficient because-

- It makes the transmission process extremely slow.

If the data packet sent by the sender gets lost, then

- Sender will keep waiting for the acknowledgement for infinite time.

- Receiver will keep waiting for the data packet for infinite time.

If acknowledgement sent by the receiver gets lost, then-

- Sender will keep waiting for the acknowledgement for infinite time.

- Receiver will keep waiting for another data packet for infinite time.

## Stop and Wait ARQ-

Stop and Wait ARQ is an improved and modified version of Stop and Wait protocol.

Stop and Wait ARQ assumes-

- The communication channel is noisy.

- Errors may get introduced in the data during the transmission.

## Working-

- Stop and wait ARQ works similar to stop and wait protocol.

- It provides a solution to all the limitations of stop and wait protocol.

- Stop and wait ARQ includes the following three extra elements.

- After sending a data packet to the receiver, sender starts the time out timer.

- If the data packet gets acknowledged before the timer expires, sender stops the time out timer.

- If the timer goes off before receiving the acknowledgement, sender retransmits the same data packet.

- After retransmission, sender resets the timer.

- This prevents the occurrence of deadlock.

- Sequence number on data packets help to solve the problem of delayed acknowledgement.

- Consider the acknowledgement sent by the receiver gets lost.

- Then, sender retransmits the same data packet after its timer goes off.

- This prevents the occurrence of deadlock.

- The sequence number on the data packet helps the receiver to identify the duplicate data packet.

- Receiver discards the duplicate packet and re-sends the same acknowledgement.

- If receiver receives a corrupted data packet from the sender, it sends a negative acknowledgement (NAK) to the sender.

- NAK requests the sender to send the data packet again.

## Limitation

- The major limitation of Stop and Wait ARQ is its very less efficiency.

## Stop and Wait Protocol Vs Stop and Wait ARQ-

The following comparison table states the differences between the two protocols-

| Stop and Wait Protocol | Stop and Wait ARQ |
| --- | --- |
| It assumes that the communication channel is perfect and noise free. | It assumes that the communication channel is imperfect and noisy. |
| Data packet sent by the sender can never get corrupt. | Data packet sent by the sender may get corrupt. |
| There is no concept of negative acknowledgements. | A negative acknowledgement is sent by the receiver if the data packet is found to be corrupt. |
| There is no concept of time out timer. | Sender starts the time out timer after sending the data packet. |
| There is no concept of sequence numbers. | Data packets and acknowledgements are numbered using sequence numbers. |

## Sliding Window Protocol-

- Sliding window protocol is a flow control protocol.

- It allows the sender to send multiple frames before needing the acknowledgements.

- Sender slides its window on receiving the acknowledgements for the sent frames.

- This allows the sender to send more frames.

- It is called so because it involves sliding of sender’s window.

- Each sending frame has to be given a unique sequence number

- The size of the sender’s window is bounded, if receiver cannot process the data fast, sender has to slow down and not transmit the frames too fast.

Maximum number of frames that sender can send without acknowledgement = Sender window size