### Media Access Control (MAC) Address –

MAC Addresses are unique 48-bits hardware number of a computer, which is embedded into network card (known as Network Interface Card) during the time of manufacturing. MAC Address is also known as Physical Address of a network device.

MAC Address is a 12-digit hexadecimal number (6-Byte binary number), which is mostly represented by Colon-Hexadecimal notation. First 6-digits (say 00:40:96) of MAC Address identifies the manufacturer, called as OUI (Organizational Unique Identifier). IEEE Registration Authority Committee assign these MAC prefixes to its registered vendors.

CC:46:D6 - Cisco

3C:5A:B4 - Google, Inc.

3C:D9:2B - Hewlett Packard

00:9A:CD - HUAWEI TECHNOLOGIES CO.,LTD

The rightmost six digits represents Network Interface Controller, which is assigned by manufacturer.

As discussed above, MAC address is represented by Colon-Hexadecimal notation. But this is just a conversion, not mandatory. MAC address can be represented using any of the following formats

### Types of MAC Address –

- Unicast – A Unicast addressed frame is only sent out to the interface leading to specific NIC. If the LSB (least significant bit) of first octet of an address is set to zero, the frame is meant to reach only one receiving NIC. MAC Address of source machine is always Unicast.

- Multicast – Multicast address allow the source to send a frame to group of devices. In Layer-2 (Ethernet) Multicast address, LSB (least significant bit) of first octet of an address is set to one

- Broadcast – Similar to Network Layer, Broadcast is also possible on underlying layer( Data Link Layer). Ethernet frames with ones in all bits of the destination address (FF-FF-FF-FF-FF-FF) are referred as broadcast address. Frames which are destined with MAC address FF-FF-FF-FF-FF-FF will reach to every computer belong to that LAN segment.