Mid-Term Exam

DEPARTMENT OF COMPUTER SCIENCE & IT

The Islamia University of Bahawalpur, Pakistan

Class: MCS (1st Semester) Mid Term					Session: 2020-2021

Subject: Digital Logic and Design 					Codes: CSIT-21102

Time: 60 Minutes						            Total Marks: 30

Name: ________________________________		            Roll. No. ________

Q 1: Encircle correct option.						(0.5x10=5)

1. When a signed binary number is represented in sign-magnitude, the left-most bit is the sign bit and the remaining bits are:

a) Sign bit		b) Magnitude bits		c) Both a & b		d) None of above

2. Mark the standard POS Expression:

a) (A' + B + C)(A + B' + C)(A + B + C)		b) (A' + C)(A + B' + C)(A + B)

c) (A' + B + C)(A + B')(A + B + C)			d) A'(A + C)(A + B + C)

3. A 4-variable Karnaugh map has:

a) eight cells		b) three cells			c) sixteen cells 	d) four cells

4. An inverter performs an operation known as:

a) Complementation	b) Assertion			c) Inversion 		d) Both a & c

5. A quantity having continuous values is

a) a digital quantity	b) an analog quantity		c) a binary quantity	d) None of above

6. The hexadecimal number system has 16 characters, six of which are:

a) Decimal numbers	b) Alphabetic characters	c) Octal numbers	d) Both a & b

7. According to Rules of Boolean Algebra (A+ B)(A + C) = _____:

a) A + B		b) A + BC			c) AB			d) None of above

8. 1’s complement of 10011001 is:

a) 01100110      	 b) 01100110			c) 01110111 		d) 01100111

9. The Domain of expression A'BCD + AB + C'D + B is :

a) A and D  		b) A, B and D			c) A, B, C and D   	d) None of these

10. The binary number 110101can be written in octal as:

a) 658			b) 768				c) 218         		d) 628

Q2: Answer the following short questions.					(2x4=8)

- Using Boolean algebra techniques, simplify the following expression as much as possible:

BD + B(D + E) + D'(D + F)

- Name two advantages of digital data as compared to analog data.

- Express this (-123) decimal number into binary as an 8-bit sign magnitude number .

- Convert this binary number to gray code: (10101111)2.

- Determine the output waveform for the NOR gate, given the input waveforms, A and B, in Figure 1.

Q 3: Long Question.	    							(7)

- Explain Sum-Of-Products(SOP) Form and also Standard SOP Form. Convert the following SOP expression into Standard SOP Form and also simplyify it by using Karnaugh Map:  AB'C + A'B' + ABC'D

- Determine the output waveform for the NAND gate, given the input waveforms, A and B, in Figure 1.

**************************