EFFLORESCENCE

Definition

When some substance are exposed to air they lose water to the atmosphere thereby reducing in weight. Solids that behave in this way are those with water of crystallization. The molecules of water of crystallization are partially or completely lost to the atmosphere

OR

The escape of water molecules from hydrated molecules of a substance is called efflorescence

Explanation

Water of crystallization

Water combined with crystals is called water of crystallization

CuSo4 combines with 5 molecules of water                         CuSo4.5H2O

MgSo4 combines with 7 molecules  of water                         MgSo4.7H2O

- The hydrated substance  under  normal conditions exerts  vapor  pressure due to its water of crystallization

- Vapor  pressure  of water is about  10mm Hg at 15c.

- When  the  vapor  pressure  of  hydrated  substance is more than 10mm Hg , the  water  escape from the crystals into the atmosphere  and the compound  form  another  solid of lower  proportion of water  or anhydrous substance this phenomena  is called efflorescence

EXPLANATION ABOUT PROCESS

- The large no of chemical  compound exists in hydrated or anhydrous  form

- In  hydrated form the water molecules attached to the compound are called the water of crystallization

- The surrounding atmosphere also has vapors and definite vapor pressure due to water in atmosphere

- If the vapor pressure  of the hydrated substance is greater than the surrounding atmosphere than the hydrated substance  loses water  molecules to become less hydrous and to make an equilibrium between the hydrated substance and the atmosphere

- Since the vapor pressure of the hydrated substance is greater then the atmospheric vapor pressure so there is a spontaneous loss of water  till the equilibrium is obtained

- After the equilibrium formation number more loss of water molecules occur

Prevention of Efflorescence

Efflorescence or loss of water of crystallization can be prevented by the following ways

- By  closing the container immediately after use

- By filling the container completely so that no space for air storage is available above the liquid in the container

EXAMPLE

When Na2Co3.10H20 slightly heated it loses 9 molecules of water and becomes monohydric

Na2Co3.H2o

DELIQUESCENCE

Deliquescence is a reverse of efflorescence.

This is when certain solid substances when exposed to air absorb water enough to form solution

EXAMPLE

solid NaOH, CaCl2, KOH, FeCl3

EXPLANATION ABOUT PROCESS

- If a hydrated substance has a lower vapor pressure  than the surrounding  atmosphere than the water molecule transfer from the atmosphere to the less hydrated substance to make them more hydrated to produce a  equilibrium

- This phenomena is called deliquescence such substances are called deliquescent  e.g. NaOH

PREVENTION OF DELIQUESCENCE

- By  closing the container immediately after using it

- By filling the container completely so that no space for  air storage is available above the liquid  in the container

- By placing the dehydrating agent e.g silica gel to absorb water

A saturated solution of CaCl 2 will exert a vapor pressure  of about 4 to 5 mmHg at ordinary temperature .

More dilute solution will exert vapor pressure up to the vapor  pressure of pure liquid.

The vapor pressure of water may rise up to 15 mmHg in atmosphere So hydrated CaCl2 will absorb the moisture  from its surrounding.

All deliquescent substance are very much soluble  in water

e.g KOH, sodium lactate and ZnCl2 etc

Many substance absorb moisture from the atmosphere but do not form liquid  these are called hygroscopic substances

In the solids by the forces responsible for adsorption many vegetables extracts and chemicals are hygroscopic e.g

Magnesium trisilicate

EXISCCATION

Definition

Exisccation is the process of removing water of crystallization from the hydrated crystalline substances and making them anhydrous .

For example

- Sodium carbonate

- Alum

- Sodium arsenate

- Ferous sulphate

- Magnesium sulphate

In the laboratory  exsiccation may be carried out by taking a weighed amount of the substance which is heated in an oven with continuous stirring until a constant weight is obtained or until the calculated loss in weight has taken place

Application

- Exsiccation is done to get an anhydrous product required in the formulation of certain medicaments

- It is also carried out to reduce the bulk and weight of certain drugs so that they can be easily administered

- Generally on exsiccation a fine powder of the substance is obtained

LYOPHILLIZATION

Freeze drying is the process of drying in which water is sublimed from the product after it is frozen.

History

Freeze drying was first actively developed during world war two transport of serum.

The main aim was to store the products without refrigeration and to remove moisture from thermolabile compounds

Atlas in 1961 built 6 production freeze drying cabinet for nestle group in Germany Holland

Principle

Lyophilization is carried out using a simple principle of physics sublimation. Sublimation is the transition of a substance from the solid to the vapour state without first passing through an intermediate liquid phase

It is performed at temperature and pressure conditions below the triple point to enable sublimation of ice

The entire process is performed at low temperature and pressure by applying vacuum hence is suited for drying of thermolabile compounds

The concentration gradient of water vapour between the drying front and condenser is the driving force for removal of water during lyophilization

Fundamental process steps are:

- Freezing: the product is frozen. This provides a necessary condition for low temperature

- Vacuum: after freezing, the product is placed under vacuum. This enables the frozen solvent in the product to vaporize without passing through liquid phase a process known as sublimation

- Heat: is applied to the frozen product to accelerate sublimation

- Condensation : low temperature condenser plates remove the vaporized solvent from the vacuum chamber by converting it back to a solid.

Process

- The freeze drying process is simple but more complicated as follows

- During the process the temperature must be less below the triple point in range of

-10c to -30c

- Similarly the process must also be kept below the triple point  (4.6 mmHg)

- The substance or solution is kept in a drying chamber

- The NH3 and Freon gases are used as refrigerants in the chambers from the compressor through a pipe

- The chamber and condenser are then evacuated by the vacuum pump

- The heat is then introduced by electric resistant coil or by circulating hot water

- The process continues until the product is dried

- The provision of heat is controlled to avoid

Freeze dry product characteristics

- Sufficient strength

- Uniform color

- Sufficient dry

- Sufficient porous

- Sterile

- Free of pyrogens and particulates

- Chemically stable both in dry state and reconstitution

Advantages of lyophilization

- Removal of water at low temperature

- Thermolabile materials can be dried

- Compatible with aseptic operations

- Sterility can be maintained

- Reconstitution is easy

Disadvantages

- Many biological molecules are damaged by the stress associated with freezing, freeze drying or both

- The product is prone to oxidation due to high porosity and large surface area, therefore the product should be packed in vacuum or using inert gas or in a container impervious to gases

- Cost may be an issue , depending on the product

- Long time process

Common lyophilized products

- Pharmaceuticals –large and small molecules

- Bacteria

- Viruses

- Vaccines

- Plasma

- Small zoological specimens

- Fruit

- Coffee

- Flowers

Application

- Pharmaceutical and biotechnology –to increase the shelf life of products , such as vaccines and other injectables

- Food

- To preserve food, very light weight.

- To produce essences or flavouring agents

- Freeze dried fruits are produced

- Culinary herbs are preserved

- Instant coffee powder is prepared

Elutriation

This is the process of separation depending upon the low density of fine particles and the high density of coarse particles. It is used to separate the fine and coarse particles present in a paste. The paste is mixed with large amount of water and the mixture is allowed to stand for some time. The heavy coarse particles settle down while the fine particles go to upward. The upper layer is separated and dried to get fine particles.

- On large scale, elutriation tanks are used.

- Chalk is produced from native calcium carbonate by elutriation.

- Kaolin is also produced by this method.

Ignition

This is a process of heating the organic substances in the excess of air until the carbon atom has burnt as CO2 remaining the residue of inorganic matter called ash.

- This process is used to determine the purities of the organic salts of alkali metals such as tartarates, citrates, benzonate and many drugs.

- The purity of a drug is determined by ash limit. When all the moisture from a drug has been removed, the drug is ignited in excess of air. During ignition drugs ash changes time to time. But at last a stage comes when there is no change in the ash. The percentage of this ash is calculated. There is a specific limit from which it should not exceed. This limit s called overall as ash limit.

Fusion

Fusion is the process by which the solid gets converted into liquids without addition of any solvent or it is also defined as the process of heating the solids until they melt.

- Fusion is done to purify certain solid and semisolid substances e.g. bees wax; hard paraffin, soft paraffin and wool fat are heated to melt and filtered while heating to remove the dissolved impurities. On cooling a product free from dissolved impurities is obtained. This method is also applied for the preparation of ointments when they contain solids and semisolids in the formulation.

- All the substances are melted and then cooled slowly with continuous stirring until a uniform product is obtained. To avoid over heating substances with higher melting points are

- melted first to which the substances with lower melting points are added.

Trituration

This method is applied when the medicament is solid and insoluble in base. So for this purpose solids are finely powdered and passed through a sieve of appropriate size

- Trituration may be carried out with an ointment tile and spatula or using a mortar and pestle.

Calcination

Calcination is the process in which the inorganic substances are strongly heated so as to remove their volatile contents and a fixed residue is obtained.

- On laboratory scale calcination is done in silica or platinum crucibles whereas in industries metallic vessels are used.

- Calcination is used in the preparation of certain inorganic substances such as calcium oxide, light magnesium oxide, heavy magnesium oxide, zinc oxide, and red mercuric oxide.

- These substances are prepared by heating their respective carbonates.

- This method is also used in gravimetric analysis of certain pharmaceutical preparations.

SUBLIMATION

Sublimation is the process in which some solids on heating give vapours which on cooling are converted to solid without passing through the liquid state. The product so obtained in known as sublimate.

Triple point.

Triple point is the point having a definite temperature and pressure at which the solid, liquid and vapor phases of a chemical substance co-exist indefinitely. OR

Triple point is a point in a graph between temperature and pressure at which three phases like solid, liquid and vapours may co exist

A solid sublimes only when the pressure on a solid is lower than the triple point. For example a solid is converted to vapors and on condensation solid is obtained; this is called “true sublimation”

But when pressure on a solid is higher than the triple point, liquid phase will be produced before conversion into vapours and then to solids, this is called “sublimation”

Method of sublimation

No specific device or apparatus is required for this process.

In laboratory it consists of a china dish, a perforated filter paper, funnel, and a cotton plug.

The material to be sublimed is placed in the china dish; the china dish is covered with a perforated filter paper. A glass funnel is placed in the inverted position over the filter paper; the open end of the funnel is plugged with cotton to prevent the escape of sublimed product.

The china dish is heated and vapours pass through the perforations of the filter paper, and are collected on the inner surface of the glass funnel which is kept cool from out side by wrapping a wet filter paper or cotton wool. The vapors on cooling are condensed into solid which is collected.

APPLICATIONS

1. This process is used in the purification of many pharmaceutical substances such as iodine, camphor, naphthaline, benzoic acid, mercuric chloride, ammonium chloride.

2. This process is also employed to purify volatile solids contaminated with non volatile impurities.

EVAPORATION

Theoretically evaporation means the free escape of vapours from the surface of a liquid below its boiling point. It is a process that can take place even at room temperature. Evaporation is too slow at room temperature so the liquid is boiled from where the vapours escape from the bulk of liquid.

- Thus evaporation may be defined as “the removal did from a solution by boiling the solution in a suitable vessel from where the vapours are withdrawn and a concentrated liquid is left behind.

- Evaporation will be maximum at the boiling point of the substance.

- Evaporation differ from boiling in that, evaporation takes place at all temperatures whereas boiling takes place only at one temperature for a given pressure.

- Evaporation occurs only at the surface of a liquid, but in boiling evaporation takes place from whole of the liquid. Boiling occurs when the vapours pressure of the liquid becomes equal to the atmospheric pressure.

FACTORS AFFECTING EVAPORATION

1) Temperature

2) Surface area

3) Agitation

4) Atmospheric aqueous pressure

5) Atmospheric pressure on the liquid under evaporation

6) Type of product required

7) Economic factors

1) Temperature

The rate of evaporation is directly proportional to the temperature.

Higher the temperature higher will be evaporation but evaporation is at its maximum at the boiling point of the liquid.Thermostable preparations can be evaporated at higher temperature but the thermolabile preparations will have to be evaporated at low temperature.

Many substances like glycosides, alkaloids, hormones, enzymes and antibiotics may get decomposed even at temperature below 100c and hence they should be evaporated at low and careful temperature. Extremely heat sensitive substances are evaporated under reduced pressure.

Time required for evaporation is very critical. Exposure to a higher temperature for a shorter period of time may be less destructive than a lower temperature with exposure for a longer period of time. Therefore as for as possible evaporation should be carried out for short time at low temperature.

Some drugs decompose more readily in the presence of moisture, especially at a raised temperature.

Similarly hydrolysis and many other degradation reactions take place in the presence of moisture.

Therefore evaporation in some cases may be carried out at a low controlled temperature till a concentrated product is obtained.

EVAPORATORS

A) Small scale methods

Small quantities of liquids can be evaporated in a glass dish. Generally direct heating by burner or electric hot plate is used for evaporation. But these should not be employed because direct heating leads to the decomposition of the substances towards the end of evaporation.

So for this purpose different types of bathes are used as a source indirect heating.

A water bath is most suitable when the liquids are to be heated up to100c.Glycerine bath is used to attain a temperature up to 150c. Sand bath or oil bath containing liquid paraffin or soft paraffin may be used when higher temperature up to 300c is required.

To prevent decomposition, whole of the liquid should not be evaporated to dryness. The last traces of the solvent from the concentrated liquid may be removed under controlled temperature.

B) Large Scale Methods

1) Evaporating pans

2) Evaporating stills

1) Evaporating pans.

On large scale, liquid extracts containing water as a menstrum are evaporated in open pans called evaporating pans. These consist of hemispherical or shallow pans made of copper, stainless steel, aluminium, enameled iron. Pans made from stainless steel and jacket from iron is most suitable. The hemispherical shape is most suitable because it provides the best surface to volume ratio for heaing.

For convenience in emptying the fixed pans, a tube passes from the lowest point of liner, to withdraw the evaporating liquid from the pan.

Advantages

1. They are simple, easy and cheap to construct.

2. Easy to use and clean.

3. Stirring of the evaporating liquid can be done easily

Disadvantages

1. The whole of the liquid is heated all the time, which may lead to the decomposition of the component

2. Solids may be deposited at the bottom, which makes the stirring necessary.

3. These pans can only be used for evaporating aqueous and thermostable liquid extracts.

4. The room in which evaporating pans are used for evaporating the liquids must be fitted with efficient exhaust fans otherwise the room will be filled with dense fog of condensed vapours and water will start falling from the roof and along the sides of the walls.

2) Evaporating stills

Evaporating stills are quite similar to evaporating pans. The vessel is made of cooper stainless steel, with a cover, which is fitted with a water condenser so that the solvent is condensed and collected in a receiver. At the lower point of the pan, a tube fitted with a tape is connected for the removal of the product from the pan. A steam jacket surrounds the pan. For cleaning the pan or easy removal of the product the cover of the vessel may be fitted with quick release system of clamps.

Advantages

- They are very simple to construct and easy to clean.

- As the vapors are condensed so it increases the rate of evaporation.

- A vacuum pump can be fitted to the apparatus by which evaporation under reduced pressure can be done

- Since the cover can be removed easily therefore the still may be conveniently used for evaporating extracts to dryness.

Disadvantages

- The whole of the liquid is to be heated all the time, which may lead to deterioration of the product.

- This method is used only for the small batches of liquids which are to be evaporated.

Applications of evaporation

- This process is used in the preparation of liquid extracts, dry extracts and in the concentration of blood serum.

- It is also used in the manufacture of drugs like antibiotics, enzymes and hormones.