# Laboratory Balance and Scale Types, Care and Terms

The history of balances and scales dates back to Ancient Egypt. A simplistic equal-arm balance on a fulcrum that compared two masses was the standard. Today, scales are much more complicated and have a multitude of uses. Applications range from laboratory weighing of chemicals to weighing of packages for shipping purposes.

To fully understand how balances and scales operate, there must be an understanding of the difference between mass and weight.

Mass is a constant unit of the amount of matter an object possesses. It stays the same no matter where the measurement is taken. The most common units for mass are the kilogram and gram.

Weight is the heaviness of an item. It is dependent on the gravity on the item multiplied by the mass, which is constant. The weight of an object on the top of a mountain will be less than the weight of the same object at the bottom due to gravity variations. A unit of measurement for weight is the newton. A newton takes into account the mass of an object and the relative gravity and gives the total force, which is weight.

Although mass and weight are two different entities, the process of determining both weight and mass is called weighing.

Balance and Scale Terms

Accuracy The ability of a scale to provide a result that is as close as possible to the actual value. The best modern balances have an accuracy of better than one part in 100 million when one-kilogram masses are compared.

Calibration The comparison between the output of a scale or balance against a standard value. Usually done with a standard known weight and adjusted so the instrument gives a reading in agreement.

Capacity The heaviest load that can be measured on the instrument.

Precision Amount of agreement between repeated measurements of the same quantity; also known as repeatability. Note: A scale can be extremely precise but not necessarily be accurate.

Readability This is the smallest division at which the scale can be read. It can vary as much as 0.1g to 0.0000001g. Readability designates the number of places after the decimal point that the scale can be read.

Tare The act of removing a known weight of an object, usually the weighing container, to zero a scale. This means that the final reading will be of the material to be weighed and will not reflect the weight of the container. Most balances allow taring to 100% of capacity.

Balance and Scale Types

Analytical Balance These are most often found in a laboratory or places where extreme sensitivity is needed for the weighing of items. Analytical balances measure mass. Chemical analysis is always based upon mass so the results are not based on gravity at a specific location, which would affect the weight. Generally capacity for an analytical balance ranges from 1 g to a few kilograms with precision and accuracy often exceeding one part in 106 at full capacity. There are several important parts to an analytical balance. A beam arrest is a mechanical device that prevents damage to the delicate internal devices when objects are being placed or removed from the pan. The pan is the area on a balance where an object is placed to be weighed. Leveling feet are adjustable legs that allow the balance to be brought to the reference position. The reference position is determined by the spirit level, leveling bubble, or plumb bob that is an integral part of the balance. Analytical balances are so sensitive that even air currents can affect the measurement. To protect against this they must be covered by a draft shield. This is a plastic or glass enclosure with doors that allows access to the pan.

Equal Arm Balance/Trip Balance This is the modern version of the ancient Egyptian scales. This scale incorporates two pans on opposite sides of a lever. It can be used in two different ways. The object to be weighed can be placed on one side and standard weights are added to the other pan until the pans are balanced. The sum of the standard weights equals the mass of the object. Another application for the scale is to place two items on each scale and adjust one side until both pans are leveled. This is convenient in applications such as balancing tubes or centrifugation where two objects must be the exact same weight.

Platform Scale This type of scale uses a system of multiplying levers. It allows a heavy object to be placed on a load bearing platform. The weight is then transmitted to a beam that can be balanced by moving a counterpoise, which is an element of the scale that counterbalances the weight on the platform. This form of scale is used for applications such as the weighing of drums or even the weighing of animals in a veterinary office.

Spring Balance This balance utilizes Hooke's Law which states that the stress in the spring is proportional to the strain. Spring balances consist of a highly elastic helical spring of hard steel suspended from a fixed point. The weighing pan is attached at the lowest point of the spring. An indicator shows the weight measurement and no manual adjustment of weights is necessary. An example of this type of balance would be the scale used in a grocery store to weigh produce.

Top-Loading Balance This is another balance used primarily in a laboratory setting. They usually can measure objects weighing around 150–5000 g. They offer less readability than an analytical balance, but allow measurements to be made quickly thus making it a more convenient choice when exact measurements are not needed. Top-loaders are also more economical than analytical balances. Modern top-loading balances are electric and give a digital readout in seconds.

Torsion Balance Measurements are based on the amount of twisting of a wire or fiber. Many microbalances and ultra-microbalances, that weigh fractional gram values, are torsion balances. A common fiber type is quartz crystal.

Triple-Beam Balance This type of balance is less sensitive than a top-loading balance. They are often used in a classroom situation because of ease of use, durability and cost. They are called triple-beam balances because they have three decades of weights that slide along individually calibrated scales. The three decades are usually in graduations of 100g, 10g and 1g. These scales offer much less readability but are adequate for many weighing applications.

Balance and Scale Care and Use

A balance has special use and care procedures just like other measuring equipment. Items to be measured should be at room temperature before weighing. A hot item will give a reading less than the actual weight due to convection currents that make the item more buoyant. And, if your balance is enclosed, warm air in the case weighs less than air of the same volume at room temperature.

Another important part of using a balance is cleaning. Scales are exposed to many chemicals that can react with the metal in the pan and corrode the surface. This will affect the accuracy of the scale.

Also, keep in mind that a potentially dangerous situation could occur if a dusting of chemicals is left on the balance pan. In many lab and classroom situations, more than one person uses a single scale for weighing. It would be impossible for each person to know what everyone else has been weighing. There is a chance that incompatible chemicals could be brought into contact if left standing or that someone could be exposed to a dangerous chemical that has not been cleaned from the balance. To avoid damaging the scale or putting others in danger, the balance should be kept extremely clean. A camel's hair brush can be used to remove any dust that can spill over during weighing.

Calibration is another care issue when it comes to scales. A scale cannot be accurate indefinitely; they must be rechecked for accuracy. There are weight sets available that allow users to calibrate the scale themselves or the scales can be calibrated by hiring a professional to calibrate them on site.

The correct weight set needs to be chosen when calibrating a scale. The classes of weight sets start from a Class One which provides the greatest precision, then to Class Two, Three, Four and F and finally go down to a Class M, which is for weights of average precision. Weight sets have class tolerance factors, and as a general rule, the tolerance factor should be greater than the readability of the scale.

Cell Membrane Stability (CMS).

Cut uppermost fully expanded leaves into 1 cm segments. Wash one gram of leaf segments into a 50 mL glass tube, slowly three times with deionized distilled water to remove surface adhered electrolytes. Submerge the washing leaf segments in 30 mL of 40 % PEG solution (PEG-6000) for 24 hours at 100C to minimize secondary effects. Adjust the concentration of the solution, duration and desired desiccation severity.  The leaf segments after the treatment period are washed quickly three times with deionized distilled water. Add 30 mL of deionized distilled water and keep for 24 hours at 10OC.  Warm the tubes at 250 C, shake well and measure the electrical conductivity by EC meter. Kill the leaf segments by autoclaving the samples for 15 minutes, warm again to 250C and measure electrical conductivity second time. Degree of membrane integrity, which permits a measure of membrane stability under stress to electrolyte leakage. Calculate the percentage injury to leaf tissue by desiccation using first and the second electrical conductivity measurements, as follows.

Percent injury = [1-(1-T1/T2) / (1- C1/C2)] x 100

T1  =  first conductivity measurement

T2 =  Second conductivity measurement

C1 =  First conductivity measurement of control

C2 =  Second conductivity measurement of control

2.	Chlorophyll contents.

Chlorophyll contents area are estimated according to the method of Arnon (1949) and Davies (1976). Cut the fresh leaves into 0.5 cm segments and extract over night with 80% acetone at 100C. Centrifuge the extract at 14000 x g for 5 minutes and the measure absorbance of the supernatant at 480, 645, 663nm.

Calculate the chlorophyll a, b total chlorophyll and carotenoids using the following formulae.

Chl a = [12.7 (OD 663) -2.69 (OD 645) ] x V/1000 x W

Chl b = [ 22.9 (OD 645) -4.68 (OD 663)] x V/1000 x W

Total Chl = [ 20.2 (OD 645) + 8.02 (OD 663)] x V/100 x W

Carotinoids (g mL-1) = Acar /Em x 100

Where Acar =OD 480 + 0.114 (OD 663) -0.638 (OD 645) E100 % cm= 2500

V = Volume of extract

W = Weight of the sample

Biochemical parameters

# 4	Glycinebetaine determination

4.1	Reagents:

4.1.1	Potassium Tri-iodide solution

Dissolve 7.5 g Iodine and 10g Potassium iodide in 100 mL of  1N HCl by continuous stirring for 30 min,  filter and store in amber colour bottle.

4.1.2	Prepare 2N HCl/H2SO4

4.1.3 	1,2 dicholroethane

4.2 	Procedure:

Grind fresh leaf material (1.0 g) in 10 mL of distilled water, filter and take 1 mL of the extract and mix 1 mL of HCl (2N).  Take 0.5 mL of mixture in a glass tube and add 0.2 mL of potassium tri-iodide solution (prepare potassium tri-iodide solution by dissolving 7.5 g iodine and 10 g potassium iodide in 100 mL of 1N HCl by continuous shaking for 30 minutes).

Shake the mixture and cool in an ice bath for 90 minutes with occasional shaking. Then add 2.0 mL of ice cooled distilled water and the 20 mL of 1-2 dichloroethane (cooled at -10oC) in the mixture. Pass  continuous stream of air for 1-2 minutes by keeping tubes still in the ice bath (4oC),  two layers are formed . Discard the upper aqueous layer and take the optical density of organic layer at 365 nm (Grieve and Gratan, 1983). The concentrations of the betaine are calculated against the standard curve. The blank is developed as above with distilled water. Develop standard curve with different concentrations of Glycine betaine.

5.	Determination of Proline

5.1	Reagents:

5.1.1	Acid Ninhyrine :

Dissolve 1.25g nin-hydrine in 20 mL glacial acetic acid and 20 mL 6M orthophosphoric acid, shake gently and mix (it is stable for 24 hours).

    - Acetic acid

    - Toluene

5.2	Procedure

The proline is estimated according to the method of the Bates et al. (1973). Homogenize fresh leaf tissue (0.5 g) in 10 mL of 3% sulfo-salicylic acid. Filter the homogenate through Whatman No. 2 filter paper. Take 2 mL of the filterate in a test tube, add 2 mL acid ninhydrin solution (1.25 g ninhydrine in 30 mL glacial acetic acid and 20 mL 6 M orthophosphoric acid), and 2 mL of glacial acetic acid, heat  for 1 h at 100oC. Terminate reaction in an ice bath. Extract reaction mixture with 10 mL toluene, mix vigorously by passing a continuous stream of air for 1-2 minutes. Aspirate toluene from chromophore.  Take aqueous phase, warm at room temperature and read at 520 nm absorbance using toluene as a blank. Determine the proline concentration from a standard curve and calculate on fresh weight basis as follows:

5.3 	Calculations

(mole proline g-1 fresh weight =

(g proline mL-1 x mL of toluene/115.5)/(g of sample/5)

6	Protein Analysis

Total soluble proteins are determined using the method of Lowry et al. (1951).

#### Reagents

    - Phosphate buffer

(0.2 M): Following chemicals are used to prepare the phosphate buffer.

1.	NaH2PO4.2H2O  molar solution (156.01 g in one liter) is prepared as the stock.

2.	Disodium hydrogen phosphate (Na2HPO4.2H2O  molar solution (177.99 g/L) is prepared as the stock).

##### Copper Reagents

###### Solution A:

Na2CO3   =	2.0 g

NaOH	    =	0.2 g

Sodium potassium tartarate =	1.0 g

All the three chemicals are dissolved in distilled water and the volume is made up to 100 mL.

###### Solution B:

CuSO4.5H2O solution = 0.5g CuSO4.5H2O is dissolved in 100 mL distilled water.

    - Solution C:

50 mL of solution A and 1.0 mL of solution B are mixed to prepare alkaline solution. This solution is always prepared fresh.

    - Folin Phenol Reagent

One hundred grams of sodium tungstate and 25 g of sodium molybdate are dissolved in 700 mL of distilled water. Fifty mL of 85% orthophosphoric acid and 100 mL of HCl are added and the mixture is refluxed for 10 h. Then 150 g of lithium sulfate is added along with 50 mL of distilled water. A few drops of Br2 are also added. The mixture is boiled without condenser for 15 min to remove extra Br2. The mixture is then cooled and diluted to 1000 mL.

    - Standard Bovine Serum Albumin (BSA) solution (1 mg/mL)

10 mg of Bovine serum albumin (BSA) is dissolved in 10.0 mL of distilled water.

## 6.1.8	Extraction

Chop / grind fresh leaf material (1.0 g) in 10 mL of phosphate buffer (0.2 M) of pH 7.0. Centrifuge the ground leaf material at 5000 x g for five min. Take supernatant for protein determination.

    - Procedure

Take 1 mL of the leaf extract in a test tube (the blank is also included along with 1 mL of phosphate buffer pH 7.0) add 1 mL of solution C mix thoroughly and allow to react for 30 min at room temperature. Then add 0.5 mL of Folin-Phenol reagent (1:1 diluted), mix well and incubate for 30 minutes at room temperature. Then read optical density (OD) at 620 nm on a spectrophotometer (Hitachi, 220, Japan).

    - calculations

Calculate total proteins as follows:

### Protein mg/mL=

OD of the sample     x     concentration of the standard  x            1

OD of standard					        mL of sample used

Total free amino acids

Total free amino acids are determined  according to Hamilton and Van Slyke, (1973).

  - Reagents:

    - Ninhydrine solution:

Dissolve 2 g ninhydrine in 100 mL distilled water

    - Pyridine solution

Make 10 % pyridine solution

  - Extract

Dry or fresh plant material (leaves) can be extracted with  phosphate buffer pH 7.

8.3	Procedure

Take  1 mL of  extract in 25 mL test tube,  add 1 mL of 10 % pyridine and 1mL of 2 % ninhydrin solution in each tube. Then heat the tubes in boiling water bath for about 30 minutes. Make the volume of each tube to 50 mL with distilled water. Read the optical density of these coloured solution at 570 nm using spectrophotometer. Develop a standard curve with Lucine and calculate free amino acids by the formulae given below:

Grraph reading of	Volume of	Dilution

Total amino acids = 	sample		     x	 sample      x	factor

(µg/g fresh wt)	-------------------------------------------------------------

Weight of fresh tissue x 1000

9	Estimation Sugars

Glucose sucrose and total sugars are determined according to Raizi et al. (1985)

  - Reagents:

9.1.1 	Prepare  5.4 N KOH.

    - O-tolidine:

Dissolve 60mL of O-tolidine (0.05 M)  and 2 g of thiourea and makeup the volume to 100 mL with glacial acetic acid.

    - Anthrone:

Disslove  150 mg Anthrone  in 100 mL H2SO4 (72% w/w).

  - Extract

Chill the fresh leaves at 00C immediately after harvest and then freeze at -400C within 10 minutes (if liquid nitrogen is not present). Grind 1g  leaves in 10 mL of 80% ethanol (v/v). Centrifuge and filter, use the supernatant for the estimation of sugars.

### Gulcose

Take 1mL of extract in 25 mL test tube, add 5mL of  O-tolidine and heat at 970C for 15min. Cool in ice cold water. Take the OD at 630 nm on spectrophotometer.

9.4	Total sugars

Take 0.1 mL ethanol extract in 25 mL test tube and add 3mL freshly prepared anthrone. Heat the mixture at 970C for 10 min. Cool the test tubes in ice bath. Read  at 625 nm on spectrophotometer.

  - Sucrose

Take 0.1mL ethanol extract in 25 mL test tube and add  0.1mL  5.4 N KOH. Heat at 970C for 10 min and cool in ice cold water then add 3 mL of anthrone heat at 97o C for 5min. Again cool in ice cold water, incubate at room temperature for 20 minutes. Read the OD at 620 nm with spectrophotometer.

    - Standard Curve

Develop a standard curve with different concentration of glucose following the above mentioned procedure and calculate sugar values by comparing samples OD with standrads.

Enzymes

10	Nitrite Reductase activity

Nitrite reductase is determined according to Ramarao et al. (1983).

10.1	Reagents:

10.1.1 	Phosphate buffer :

Prepare 0.2 M phosphate buffer pH 5.0

10.1.2 	Sodium nitrite solution:

Prepare 0.2 M sodium nitrite solution in phosphate buffer of pH 5.0

10.1.3 	Sulphanilamide:  Dissolve 1 g of sulphanilamide in 100 mL of 2N HCl

10.1.4	N(1-Naphthyl)-ethylene diamine dihydrochloride solution:

Dissolve 0.02 g of  N(1-Naphthyl)-ethylene diamine dihydrochloride in 100 mL of distilled water.

10.2 	Procedure:

Chop 0.5 g of leaves in 25 mL test tube containing 4.5 mL of phosphate buffer (pH 5.0) and add 0.5mL of NaNO2 (0.02 M). Evacuate the tubes with a pump for 2 min and then release the vacuum. Then wrap the test tubes in aluminum foil and incubate at 300C in a water bath with gentle shaking for 30 minutes. Then transfer the tubes in boiling water for 2 min to terminate the reaction and cool the test tubes.

10.3 	Assay

Take 1mL of above extract and add 0.5 mL of 1% sulphanilamide in 2N HCl and 0.5 mL of 0.02% aqueous solution of N (1-Naphthyl)-ethylene diamine dihydrochloride. Allow the color to develop for 20 minutes. Make the total volume up to 10 mL with distilled water and read at 540 nm on spectrophotometer. Develop the standard curve with NaNO2. Follow all above procedure with substrate NO2 and blank containing no NaNO2. The activity is expressed as presence of µmol NO2 g-1 fresh weight h-1.

11.	Nitrate Reductase activity.

Nitrate reductase is estimated according to Sym (1984).

  - Reagents:

    - Phosphate buffer (pH 7.0) : Prepare 0.02 M phosphate buffer of pH 7 by dissolving sodium dihydrogen phosphate and disodium hydrogen phosphate containing 0.1 % triton X-100.

    - Substrate KNO3 solution: Prepare 0.02 M KNO3 solution in phosphate buffer of pH 7.0.

    - Sulphanilamide:  Dissolve 1 g of sulphanilamide in 100 mL of 2N HCl.

11.1.4	N(1-Naphthyl)-ethylene diamine dihydrochloride solution: Dissolve 0.02 g of  N(1-Naphthyl)-ethylene diamine dihydrochloride in 100 mL of distilled water.

  - Procedure

Chop the leaf samples (0.5 g) about 0.5 cm in length and widthwise in a 25 mL test tube containing 4.5 mL of  0.02 M phosphate buffer of pH 7.0 and add 0.5 mL of 0.02 M KNO3. Incubate these test tubes in dark at 320C for 1 h. Assay through nitrite estimation in the reaction medium where NO3 is converted into NO2 by the Nitrate reductase.

  - Assay:

After  1 h incubation period, take 1 mL of reaction medium and mix with 0.5 mL of sulphanilamide. Add immediately 0.5 mL of N(1-naphthyl)-ethylene diamine dihydrochloride after shaking. Pink diazo coloured complex with NO2 is produced. After 20 minutes dilute colour with 5mL distilled water and centrifuge for 5 minutes at 2000 rpm to remove turbidity if present. Read the absorbance at 542 nm against a set of standards developed with NaNO2. Use blank consisting of 1 mL of incubated  buffer  with all chemicals.

Peroxidase activity

Estimation of peroxidase activity according to Kuroda et al. (1990)

  - Reagents

15.1.1	Sodium phosphate buffer 50 mM having pH 7.0 containing 0.1% (V/V) Triton x-100.

15.1.2 Guaiacul 3.4 mM containing 0.9 mM H2O2, and 50 mM. Sodium phosphate buffer (pH. 6.0).

  - Enzyme extract:

Homogenize 1 g leaves using mortar and pestle in 50 mM sodium phosphate buffer (pH 7.0) containing 0.1% (v/v) Triton X-100. Centrifuge the homogenate for 15 minutes at 23000 rpm. Use supernatant for estimation of peroxidase activity.

  - Procedure:

Take 1 mL of supernatant and add 1 mL of  solution containing 3.4 mM guaiacal 0.9 mM H2O2 prepared in  50 mM phosphate buffer  (pH 6.0), incubate for 1 h at room temperature. Measure the increase in absorbance at 470 nm with respect to blank not containing enzyme extract. Note increase in OD by running a blank sample and activity.

    - Calculation:

Calculate the activity using an extension coefficient of 26.6 mM  mL-1 of tetra guacoal. (1 O.D.=26.6). Express the activity as m mol of tetra guacoal per g fresh weight per hour.

- Nutrients Analysis

  - Digestion

To digest dried ground material, take 0.5 g of plant material in digestion tubes, add 5 mL of concentrated H2SO4  (Wolf, 1982). Incubate all the tubes overnight at room temperature. Then add 0.5 mL of H2O2 (35%) and transfer the tubes in a digestion block and heat at 350oC upto the production of fumes. Heat continuously for another 30 minutes.  Remove the digestion tubes from the block and cool for 5 minutes. Add  0.5 mL of H2O2 slowly and place the tubes back into the digestion block.  Repeat the above step until the colour of  digested material is colorless. Make the volume of the extract up to 50 mL in volumetric flasks. Filter the extract and use it for the determination of K, Ca, P, N other macro and micro nutrients.

- Determination of Cations (Na+, K+ and Ca+2)

Sodium (Na+), potassium (K+) and calcium (Ca+2) are analyzed on flame photometer (Jenway PFP 7).

  - Estimation of Nitrogen

Nitrogen is estimated by micro–Kjeldahls  method (Bremner, 1965).

26.1.1	Reagents:

26.1.2	Boric acid solution (2 %)

26.1.3	Sulphuric acid, standard (0.01 N)

26.1.4	Mixed indicator of bromocresol green and methylene red

#### 26.2     Procedure

Take 5 mL of the above aliquot in Kjeldahl flask, place it on the Kjeldahl ammonia distillation unit, add 5 mL of 40% NaOH. Take 5 mL of  boric acid solution in a conical flask with few drops of mixed indicator. When the distillate is approximately 40 mL, stop the distillation.  Cool the distillate for few minutes and titrate it with 0.01 N  standard  H2SO4 till the solution turns pink. Run a blank for the complete procedure.

Estimate N by the following formula;

N %age = (V2-V1) x N x 0.014 x 100

W

Where

V2	=	Volume of standard H2SO4 required to titrate the sample solution.

V1	=	Volume of standard H2SO4 required to titrate the blank solution.

N	=	Normality of H2SO4

W	=	Weight of the sample in g

- Phosphorus

27.1.	Barton’s Reagent

27.2`	Solution A:

Dissolve 25 g of ammonium molybdate  in 400 mL of distilled water.

  - Solution B:

Dissolve 1.25 g of ammonium metavandate in 300 mL of boiling water, cool and add 250 mL of conc. HNO3. Cool the solution again at room temperature.

Mix solution A and B and make its volume up to one liter. It can be stored at room temperature.

  - Procedure:

Determination of phosphorus (P) according to Wolf (1982) / (Jackson, 1962) by spectrophotometer. Take 5 mL aliquot  and dissolve  5 mL of Barton’s reagent and make volume upto 50 mL. Let these samples stand for half an hour at room temperature before taking O.D at 470 nm.

Calculate the values of phosphorus by using standard curve. The Barton’s reagent is prepared as described above:

0, 2, 4, 6, 8, 10 ppm standards of P are prepared from sodium dihydrogen phosphate and colour is developed with Barton’s reagent and read at 470 nm. Prepare a standard curve with these and read the OD of the samples against concentration from the standard curve.